END PROJECT

BEGIN PROJECT

GEODETIC SURVEY CONTROL PROJECT (DV440-07)

DAVIDSON COUNTY

I-440

From: I-40 west of Nashville To: I-24 east of Nashville

April 7, 2017

Station             North(ft)            East(ft)          Elevation(ft)     D.A. Factor    Horizontal Datum    Vertical Datum

19-040-46     664423.1622    1725444.1366      613.277             1.00006         NAD83 (1995)         NAVD 1988

19-440-36     663873.2475    1725632.0918      630.183             1.00006         NAD83 (1995)         NAVD 1988

19-440-37     663564.6738    1725658.7489      631.074             1.00006         NAD83 (1995)         NAVD 1988

19-440-38     663032.3464    1725665.2426      620.755             1.00006         NAD83 (1995)         NAVD 1988

Station             North(ft)            East(ft)          Elevation(ft)     D.A. Factor    Horizontal Datum    Vertical Datum

19-440-08    662892.1191    1725648.4655      619.314             1.00006         NAD83 (1995)         NAVD 1988

19-440-09    662340.0803    1725670.0660      606.204             1.00006         NAD83 (1995)         NAVD 1988

19-440-39    661737.4359    1725727.1154      594.295             1.00006         NAD83 (1995)         NAVD 1988

19-440-40    661308.7238    1725777.5970      589.075             1.00006         NAD83 (1995)         NAVD 1988

19-440-41    661028.0462    1725789.9069      582.663             1.00006         NAD83 (1995)         NAVD 1988

19-440-42    660493.4708    1725748.4318      569.263             1.00006         NAD83 (1995)         NAVD 1988

19-440-27    659776.9552    1725631.7135      547.287             1.00006         NAD83 (1995)         NAVD 1988

19-440-28    659261.4414    1725505.0245      543.347             1.00006         NAD83 (1995)         NAVD 1988

19-440-29    658707.8530    1725395.3829      562.631             1.00006         NAD83 (1995)         NAVD 1988

19-440-43    658689.2440    1725557.1488      543.345             1.00006         NAD83 (1995)         NAVD 1988

19-440-10    657991.2466    1725752.5923      546.376             1.00006         NAD83 (1995)         NAVD 1988

19-440-11    657188.8494    1726171.1248      550.341             1.00006         NAD83 (1995)         NAVD 1988

19-440-44    656037.5467    1726737.4750      556.992             1.00006         NAD83 (1995)         NAVD 1988

19-440-45    654970.3347    1727312.9273      550.618             1.00006         NAD83 (1995)         NAVD 1988

19-440-46    664423.1696    1725444.2054      613.280             1.00006         NAD83 (1995)         NAVD 1988

19-440-47    652892.0192    1728652.9717     570.074              1.00006         NAD83 (1995)         NAVD 1988

19-440-48    652644.8137    1729633.7414      567.869             1.00006         NAD83 (1995)         NAVD 1988

19-440-49    652591.1704    1729865.6972      564.173             1.00006         NAD83 (1995)         NAVD 1988

19-440-12    652294.1135    1730623.5250      564.971             1.00006         NAD83 (1995)         NAVD 1988

19-440-13    652036.7441    1731005.6116      570.693             1.00006         NAD83 (1995)         NAVD 1988

19-440-50    651594.7890    1731356.8393      581.284             1.00006         NAD83 (1995)         NAVD 1988

19-440-51    650957.4764    1731815.8067      580.120             1.00006         NAD83 (1995)         NAVD 1988

19-440-52    650230.8917    1732370.0419      556.466             1.00006         NAD83 (1995)         NAVD 1988

19-440-53    649822.7668    1733546.9492      532.983             1.00006         NAD83 (1995)         NAVD 1988

19-440-54    649868.9947    1734303.1225      526.552             1.00006         NAD83 (1995)         NAVD 1988

Station             North(ft)            East(ft)          Elevation(ft)     D.A. Factor    Horizontal Datum    Vertical Datum

19-440-14    649874.6658    1734877.1254      526.249             1.00006         NAD83 (1995)         NAVD 1988

19-440-15    649868.4120    1735285.6367      526.070             1.00006         NAD83 (1995)         NAVD 1988

19-440-55    649863.2097    1735606.6455      527.305             1.00006         NAD83 (1995)         NAVD 1988

19-440-56    649808.3099    1736768.9906      530.659             1.00006         NAD83 (1995)         NAVD 1988

19-440-16    649732.8670    1738375.9711      508.753             1.00006         NAD83 (1995)         NAVD 1988

19-440-17    649676.7008    1739499.3659      547.961             1.00006         NAD83 (1995)         NAVD 1988

19-440-57    649417.0229    1739385.4282      511.945             1.00006         NAD83 (1995)         NAVD 1988

19-440-58    648955.9907    1740029.7293      517.604             1.00006         NAD83 (1995)         NAVD 1988

19-440-59    649708.3805    1739700.0951      557.555             1.00006         NAD83 (1995)         NAVD 1988

19-440-60    648459.8490    1740688.9250      510.919             1.00006         NAD83 (1995)         NAVD 1988

19-440-61    649455.3322    1740493.4080      527.739             1.00006         NAD83 (1995)         NAVD 1988

19-440-62    649783.1483    1740863.2255      527.472             1.00006         NAD83 (1995)         NAVD 1988

19-440-19    650354.2334    1741495.7572      522.182             1.00006         NAD83 (1995)         NAVD 1988

19-440-63    651123.8454    1742312.6536      525.142             1.00006         NAD83 (1995)         NAVD 1988

19-440-64    651446.8684    1742635.6386      521.901             1.00006         NAD83 (1995)         NAVD 1988

19-440-65    651736.5808    1743108.6546      516.027             1.00006         NAD83 (1995)         NAVD 1988

19-440-66    651843.4388    1743434.3229      519.335             1.00006         NAD83 (1995)         NAVD 1988

19-440-67    651883.6763    1743754.4556      527.783             1.00006         NAD83 (1995)         NAVD 1988

19-440-68    651870.1140    1744051.1130      536.727             1.00006         NAD83 (1995)         NAVD 1988

19-440-69    651742.3913    1745096.8783      567.718             1.00006         NAD83 (1995)         NAVD 1988

19-440-70    651887.5115    1745874.2566      567.471             1.00006         NAD83 (1995)         NAVD 1988

19-440-20    651978.2161    1746068.1978      562.781             1.00006         NAD83 (1995)         NAVD 1988

19-440-71    652095.0774    1746401.8447      556.704             1.00006         NAD83 (1995)         NAVD 1988

19-440-72    652401.3199    1747219.4503      532.548             1.00006         NAD83 (1995)         NAVD 1988

19-440-21    652444.5037    1747332.3779      531.837             1.00006         NAD83 (1995)         NAVD 1988

Station             North(ft)            East(ft)          Elevation(ft)     D.A. Factor    Horizontal Datum    Vertical Datum

19-440-73    652530.1980    1747758.2275      527.761             1.00006         NAD83 (1995)         NAVD 1988

19-440-74    652634.2988    1748336.7490      524.334             1.00006         NAD83 (1995)         NAVD 1988

19-440-75    652684.9889    1748760.2839      521.662             1.00006         NAD83 (1995)         NAVD 1988

19-440-76    652764.5013    1750435.3976      522.564             1.00006         NAD83 (1995)         NAVD 1988

19-440-77    652885.8786    1751330.5652      531.706             1.00006         NAD83 (1995)         NAVD 1988

19-440-22    653318.7294    1752894.9994      552.405             1.00006          NAD 83 (1995)       NAVD 1988

19-440-07    653399.1134    1753125.1213      559.261             1.00006          NAD 83 (1995)       NAVD 1988

19-440-78    664384.8257    1723778.1419      575.505             1.00006         NAD83 (1995)         NAVD 1988

19-440-79    664706.9679    1724475.9190      600.431             1.00006         NAD83 (1995)         NAVD 1988

19-440-80    664821.5912    1724887.4516      613.521             1.00006         NAD83 (1995)         NAVD 1988

19-440-81    664549.6364    1725543.5319      602.477             1.00006         NAD83 (1995)         NAVD 1988

19-440-82    664778.0191    1725571.9918      612.553             1.00006         NAD83 (1995)         NAVD 1988

19-440-83    664650.5086    1725568.3311      618.419             1.00006         NAD83 (1995)         NAVD 1988

19-440-84    665235.1698    1725864.5522      571.597             1.00006         NAD83 (1995)         NAVD 1988

19-440-85    664744.4861    1725819.7351      584.059             1.00006         NAD83 (1995)         NAVD 1988

19-440-86    664184.3724    1725754.6397      589.409             1.00006         NAD83 (1995)         NAVD 1988

19-440-87    663704.0126    1725808.0182      596.017             1.00006         NAD83 (1995)         NAVD 1988

19-440-88    663204.7233    1725801.8371      611.884             1.00006         NAD83 (1995)         NAVD 1988

19-440-89    662520.4852    1725807.7832      607.851             1.00006         NAD83 (1995)         NAVD 1988

19-440-90    661776.2587    1725869.6170      594.637             1.00006         NAD83 (1995)         NAVD 1988

19-440-91    661028.3488    1725936.6624      587.451             1.00006         NAD83 (1995)         NAVD 1988

19-440-92    660367.3628    1725895.0090      567.091             1.00006         NAD83 (1995)         NAVD 1988

19-440-93    659429.9841    1725726.1705      539.344             1.00006         NAD83 (1995)         NAVD 1988

19-440-94    658636.6876    1725922.6952      573.368             1.00006         NAD83 (1995)         NAVD 1988

Station             North(ft)            East(ft)          Elevation(ft)     D.A. Factor    Horizontal Datum    Vertical Datum

19-440-95    658683.5458    1725704.8530      540.058              1.00006         NAD83 (1995)         NAVD 1988

19-440-96    658426.0516    1725753.6803      541.157              1.00006         NAD83 (1995)         NAVD 1988

19-440-97    657959.5954    1725916.8980      544.880              1.00006         NAD83 (1995)         NAVD 1988

19-440-98    656839.6844    1726507.0433      552.805              1.00006         NAD83 (1995)         NAVD 1988

19-440-99    655891.0007    1727002.5863      556.800              1.00006         NAD83 (1995)         NAVD 1988

19-440-100    654829.1412    1727555.5569      549.600            1.00006         NAD83 (1995)         NAVD 1988

19-440-101    654192.5665    1727885.6287      544.751            1.00006         NAD83 (1995)         NAVD 1988

19-440-102    653706.3623    1728132.2495      541.630            1.00006         NAD83 (1995)         NAVD 1988

19-440-103    653324.3135    1728396.8686      550.328            1.00006         NAD83 (1995)         NAVD 1988

19-440-104    653081.4221    1728683.6703      560.232            1.00006         NAD83 (1995)         NAVD 1988

19-440-105    652939.1695    1729024.4850      566.842            1.00006         NAD83 (1995)         NAVD 1988

19-440-106    652804.0734    1729483.5856      570.218            1.00006         NAD83 (1995)         NAVD 1988

19-440-107    652629.2440    1730189.3120      561.457            1.00006         NAD83 (1995)         NAVD 1988

19-440-32     652042.8328    1731266.7713      579.483             1.00006         NAD83 (1995)         NAVD 1988

19-440-31     651302.2189    1731783.7350      582.284             1.00006         NAD83 (1995)         NAVD 1988

19-440-30      650876.7490    1732053.7221      576.411            1.00006         NAD83 (1995)         NAVD 1988

19-440-108    650494.4651    1732333.7818      561.482            1.00006         NAD83 (1995)         NAVD 1988

19-440-109    650279.4996    1732540.8906      547.667            1.00006         NAD83 (1995)         NAVD 1988

19-440-110    650122.0945    1732786.7541      537.835            1.00006         NAD83 (1995)         NAVD 1988

19-440-111    650007.1704    1733111.8408      533.017            1.00006         NAD83 (1995)         NAVD 1988

19-440-112    649975.1130    1733385.0935      531.295            1.00006         NAD83 (1995)         NAVD 1988

19-440-113    650018.1532    1734274.7770      528.030            1.00006         NAD83 (1995)         NAVD 1988

19-440-114    650012.4323    1735422.0875      527.138            1.00006         NAD83 (1995)         NAVD 1988

19-440-115    649972.4770    1736080.3311      529.761            1.00006         NAD83 (1995)         NAVD 1988

19-440-116    649928.1015    1737039.6620      527.556            1.00006         NAD83 (1995)         NAVD 1988

Station             North(ft)            East(ft)          Elevation(ft)     D.A. Factor    Horizontal Datum    Vertical Datum

19-440-117    649862.9730    1738305.9977      508.532           1.00006         NAD83 (1995)         NAVD 1988

19-440-118    650044.1120    1739320.5028      499.230           1.00006         NAD83 (1995)         NAVD 1988

19-440-119    650362.9629    1739737.3598      503.089           1.00006         NAD83 (1995)         NAVD 1988

19-440-120    651052.6703    1739923.1022      503.474           1.00006         NAD83 (1995)         NAVD 1988

19-440-121    649959.8507    1739878.6148      503.599           1.00006         NAD83 (1995)         NAVD 1988

19-440-122    649823.3154    1739528.1179      548.716           1.00006         NAD83 (1995)         NAVD 1988

19-440-123    650156.2249    1740907.4120      540.311           1.00006         NAD83 (1995)         NAVD 1988

19-440-124    650935.6221    1740383.7658      507.742           1.00006         NAD83 (1995)         NAVD 1988

19-440-125    650759.9506    1740437.9492      514.707           1.00006         NAD83 (1995)         NAVD 1988

19-440-126    650509.1730    1740724.8041      517.646           1.00006         NAD83 (1995)         NAVD 1988

19-440-127    650420.7275    1741109.7131      513.275           1.00006         NAD83 (1995)         NAVD 1988

19-440-128    650643.4031    1741555.6845      511.989           1.00006         NAD83 (1995)         NAVD 1988

19-440-129    650920.2016    1741900.2063      518.546           1.00006         NAD83 (1995)         NAVD 1988

19-440-130    651175.9624    1742162.2896      522.642           1.00006         NAD83 (1995)         NAVD 1988

19-440-131    651590.5854    1742595.1368      528.312           1.00006         NAD83 (1995)         NAVD 1988

19-440-132    651976.4831    1743376.1199      525.883           1.00006         NAD83 (1995)         NAVD 1988

19-440-133    652024.2674    1743956.6217      540.588           1.00006         NAD83 (1995)         NAVD 1988

19-440-134    651904.3202    1744917.6319      558.183           1.00006         NAD83 (1995)         NAVD 1988

19-440-135    651905.2139    1745248.7623      562.067           1.00006         NAD83 (1995)         NAVD 1988

19-440-136    651964.2291    1745576.2352      563.597           1.00006         NAD83 (1995)         NAVD 1988

19-440-137    652221.7422    1746292.3840      558.206           1.00006         NAD83 (1995)         NAVD 1988

19-440-138    652516.8697    1747057.6982      538.666           1.00006         NAD83 (1995)         NAVD 1988

19-440-139    652633.1168    1747444.3699      534.259           1.00006         NAD83 (1995)         NAVD 1988

19-440-140    652758.6359    1748150.2166      528.556           1.00006         NAD83 (1995)         NAVD 1988

19-440-141    652840.3437    1749093.9402      519.673           1.00006         NAD83 (1995)         NAVD 1988

Station             North(ft)            East(ft)          Elevation(ft)     D.A. Factor    Horizontal Datum    Vertical Datum

19-440-142    652879.8438    1750234.1787      519.186           1.00006         NAD83 (1995)         NAVD 1988

19-440-143    652928.9731    1750804.7954      524.346           1.00006         NAD83 (1995)         NAVD 1988

19-440-144    653033.2652    1751400.7294      529.510           1.00006         NAD83 (1995)         NAVD 1988

19-440-145    653216.7844    1751979.2833      539.496           1.00006         NAD83 (1995)         NAVD 1988

19-440-146    653410.6612    1752467.8781      543.755           1.00006         NAD83 (1995)         NAVD 1988

19-440-147    653631.9761    1752914.2661      538.078           1.00006         NAD83 (1995)         NAVD 1988

19-440-148    653978.1318    1753262.6384      523.298           1.00006         NAD83 (1995)         NAVD 1988

19-440-149    654380.9680    1753465.5698      506.551           1.00006         NAD83 (1995)         NAVD 1988

PROJECT LOCATION PHOTOS

Geodetic Survey Control Data

County:  DAVIDSON		Year:  2010		Point Name: 19-040-46

Latitude(N):  36°09'20.56210"	             Longitude(W):  86°49'25.20427"

D.A. Northing:         664423.1696 (Ft)

D.A. Easting:         1725444.2054 (Ft)

Elevation:                    613.280 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:  1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-040-46 driven flush

Route Description:  From the junction of I-40 and S.R. 155 (Briley Pkwy./ Robertson Rd.), west of Nashville, go east on I-40 for 1.85 miles to the entrance ramp to I-440 eastbound, continue ahead for 0.1 mile to the station on the left.

References: 2.6 ft SW of an orange WITNESS POST  7.3 ft NW of a METAL SIGN (Right Curve)  14.6 ft NE of the eastbound entrance ramp to I-440  103.6 ft NW of the end of GAURDRAIL

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-36

Latitude(N): 36°09'15.140051"	             Longitude(W): 86°49'22.856111"

D.A. Northing:                663873.2475 (Ft)

D.A. Easting:                 1725632.0918 (Ft)

Elevation:                          630.183 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 0.2 miles to the station on the right (in the concrete shoulder).

References: 8.2 ft W of the FOGLINE of I-440 eastbound  17.6 ft S of the end of GUARDRAIL  30.1 ft  SE of the OVERHEAD SIGN POLE (Entering I-440 Parkway)  41.7 ft E of an orange WITNESS POST

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-37

Latitude(N): 36°09'12.091017"	             Longitude(W): 86°49'22.499445"

D.A. Northing:                663564.6738 (Ft)

D.A. Easting:                1725658.7489 (Ft)

Elevation:                          631.074 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 0.25 miles to the station on the right (in the concrete shoulder before the concrete bridge).

References: 1.1 ft E of the front of the GUARDRAIL, 2.3 ft E of witness post at the GUARDRAIL POST, 11.3 ft W of the FOGLINE of I440 Eastbound, 17.2 ft NNE of the NW CORNER OF THE CONCRETE BRIDGE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-38

Latitude(N): 36°09'06.827776"	             Longitude(W): 86°49'22.365695"

D.A. Northing:               663032.3464 (Ft)

D.A. Easting:                1725665.2426 (Ft)

Elevation:                          620.755 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 0.35 miles to the station on the right (in the concrete shoulder).

References: 1.9 ft E of the BASE OF THE CONCRETE JERSEY BARRIER, 9.9 ft W of the FOGLINE of I440 Eastbound, 13 ft SE of the FIBER OPTIC WARNING POST, 14.5 ft E of the WITNESS POST AT THE CONCRETE WALL, 54.1 ft NE of the OVERHEAD SIGN POLE (exit 1/exit 1A).

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson 		Year: 2011		Point Name: 19-440-08

Latitude(N):  36°09'05.439779"	             Longitude(W): 86°49'22.555875"

D.A. Northing:              662892.1191 (Ft)

D.A. Easting:               1725648.4655 (Ft)

Elevation:                         619.314 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-08 driven flush

Route Description:  From the junction of I-440 east and I-40 west, just west-southwest of downtown Nashville, go east on I-440 for 0.3 mile to the station on the right.

References: 12.0 ft E of an orange WITNESS POST  13.1 ft E of a CONCRETE WALL  26.3 ft W of the FOGLINE of I-440  77.0 ft SSW of the end of GAURDRAIL

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-39

Latitude(N): 36°08'54.028524"	             Longitude(W): 86°49'21.478591"

D.A. Northing:               661737.4359 (Ft)

D.A. Easting:                1725727.1154 (Ft)

Elevation:                          594.295 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-39 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 0.6 miles to the station on the right (in the concrete shoulder).

References: 1.4 ft W of BEHIND THE GUARDRAIL, 13.1 ft NE of the OVERHEAD SIGN POLE (exit 1/exit 1A), 13.5 ft W of the FOGLINE OF I440 Eastbound, 23.5 ft of the WITNESS POST AT THE CONCRETE WALL.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-40

Latitude(N): 36°08'49.793506"	             Longitude(W): 86°49'20.819192"

D.A. Northing:                   661308.7238 (Ft)

D.A. Easting:                    1725777.5970 (Ft)

Elevation:                             589.075 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 0.05 miles to the station on the right (in the concrete shoulder).

References: 4.4 ft W of the FOGLINE of I440 Eastbound, 10 ft E of the WITNESS POST BEHIND THE CONCRETE BARRIER, 10 ft E of the END OF THE CONCRETE WALL, 38.5 ft SE of the SW CORNER OF THE CONCRETE BRIDGE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-41

Latitude(N): 36°08'47.019117"	             Longitude(W): 86°49'20.640355"

D.A. Northing:              661028.0462 (Ft)

D.A. Easting:               1725789.9069 (Ft)

Elevation:                            582.663 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 0.72 miles to the station on the right (in the concrete shoulder).

References: 12 ft W of the FOGLINE of I440 Eastbound, 33.8 ft E of the WITNESS POST AT THE CONCRETE WALL, 47.9 ft S of the END OF THE CONCRETE BARRIER, 134.5 ft N of the SINGLE LIGHT POLE #HJ38.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-42

Latitude(N): 36°08'41.729650"	             Longitude(W): 86°49'21.091233"

D.A. Northing:               660493.4708 (Ft)

D.A. Easting:                1725748.4318 (Ft)

Elevation:                          569.263 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped19-440-42 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 0.83 miles to the station on the right (in the concrete shoulder).

References: 13.3 ft E of the WITNESS POST AT ROCK BLUFF, 16.4 ft W of the FOGLINE OF I440 Eastbound, 61.6 ft NE of EXIT 1A/ AQUANIS COLLEGE SIGN, 79.7 ft NNE of the SINGLE LIGHT POLE #HJ36, 80.2 ft S of SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-43

Latitude(N): 36°08'23.873100"	             Longitude(W): 86°49'23.238056"

D.A. Northing:              658689.2440 (Ft)

D.A. Easting:               1725557.1488 (Ft)

Elevation:                         543.345 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-43 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 1.2 miles to the station on the right (in the concrete shoulder).

References: 8.3 ft E of the WITNESS POST AT WINGWALL, 17.7 ft NE of the FIBER OPTIC WARNING POST, 18.3 ft SE of the END OF WINGWALL, 21.7 ft W of the FOGLINE OF I440 Eastbound.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-44

Latitude(N): 36°07'57.750256"	             Longitude(W): 86°49'08.578634"

D.A. Northing:                  656037.5467 (Ft)

D.A. Easting:                   1726737.4750 (Ft)

Elevation:                            556.992 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-44 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 1.72 miles to the station on the right (in the concrete shoulder).

References: 13.1 ft SE of the end of the GUARDRAIL, 13.3 ft SW of the FOGLINE of I440 Eastbound, 16.9 ft NE of the WITNESS POST AT ROCK BLUFF, 39 ft SE of TRAFFIC CAMERA POLE, 72.2 ft NW of SINGLE LIGHT POLE #HI26.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-45

Latitude(N): 36°07'47.244909"	             Longitude(W): 86°49'01.455461"

D.A. Northing:             654970.3347 (Ft)

D.A. Easting:              1727312.9273 (Ft)

Elevation:                       550.618 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-45 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 1.92 miles to the station on the right (in the concrete shoulder).

References 3.8 ft SE of the end of the GUARDRAIL, 13.4 ft ENE of the WITNESS POST AT ROCK BLUFF, 15.8 ft WSW of the FOGLINE of I440 Eastbound, 32.3 ft SE of POST FOR THE DIGITAL OVERHEAD SIGN, 83.2 ft NNW of SINGLE LIGHT POLE # HI20.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-46

Latitude(N): 36°07'36.134839	             Longitude(W): 86°48'54.152051"

D.A. Northing:                 664423.1696 (Ft)

D.A. Easting:                  1725444.2054 (Ft)

Elevation:                            613.280 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-46 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 2.17 miles to the station on the right (in the concrete shoulder).

References: 13.9 ft SW of the FOGLINE of I440 Eastbound, 16.6 ft NE of WITNESS POST AT CONCRETE WALL, 18.6 ft ESE of SINGLE LIGHT POLE  #HI15, 28.2 ft N of the FIBER OPTIC WARNING POST, 60.2 ft SE of  the END OF THE CONCRETE BARRIER.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-47

Latitude(N): 36°07'26.804192"	             Longitude(W): 86°48'44.911062"

D.A. Northing:                 652892.0192 (Ft)

D.A. Easting:                  1728652.9717 (Ft)

Elevation:                            570.074 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-47 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 2.4 miles to the station on the right (in the concrete shoulder).

References: 13.8 ft NNE of the WITNESS POST AT THE CONCRETE WALL, 16 ft SSW of the FOGLINE of I440 Eastbound, 40.9 ft WNW of SINGLE LIGHT POLE, 146.7 ft ESE of SINGLE LIGHT POLE #HI11.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-48

Latitude(N): 36°07'24.44009"	             Longitude(W): 86°48'32.93219"

D.A. Northing:                 652644.8136 (Ft)

D.A. Easting:                 1729633.7414 (Ft)

Elevation:                           567.869 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 2.6 miles to the station on the right (in the concrete shoulder).

References: 2.9 ft N of BASE OF CONCRETE BARRIER, 9.4 ft S of FOGLINE OF I440 Eastbound, 11.6 ft NE of a A LIGHT POLE (w/double  lights).

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-49

Latitude(N): 36°07'23.928606"	             Longitude(W): 86°48'30.099658"

D.A. Northing:               652591.1704 (Ft)

D.A. Easting:               1729865.6972 (Ft)

Elevation:                          564.173 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 2.65 miles to the station on the right (in the concrete shoulder).

References: 3.1 ft NNE of BASE OF CONCRETE BARRIER, 9.2 ft SSW of FOGLINE of I440 Eastbound, 31 ft ENE of A LIGHT POLE (w/ double lights).

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-50

Latitude(N): 36°07'14.197463"	             Longitude(W): 86°48'11.825601"

D.A. Northing:                   651594.7890 (Ft)

D.A. Easting:                    1731356.8393 (Ft)

Elevation:                              581.284 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-50 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 3.0  miles to the station on the right (in the concrete shoulder).

References: 5.2 ft N of WITNESS POST AT SINGLE LIGHT POST #HG24, 13.6 ft S of FOGINE of I440 Eastbound, 142.7 ft W of SINGLE LIGHT POST #HG23.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-51

Latitude(N): 36°07'07.932713"	             Longitude(W): 86°48'06.168161"

D.A. Northing:                   650957.4444 (Ft)

D.A. Easting:                    1731815.8100 (Ft)

Elevation:                             580.120 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-51 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 3.15 miles to the station on the right (in the concrete shoulder).

References: 15.3 ft NE of WITNESS POST AT COVE WALL, 16.7 ft SW of FOGLINE of I440 Eastbound, 30.4 ft E of FIBER OPTIC WARNING POST, 46.5 ft N of A SINGLE POST, 68.2 ft NW of A SINGLE LIGHT POST #HF29.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-52

Latitude(N): 36°07'00.792833"	             Longitude(W): 86°47'59.341022"

D.A. Northing:                650230.8917 (Ft)

D.A. Easting:                 1732370.0419 (Ft)

Elevation:                            556.466 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-52 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 3.3 miles to the station on the right (in the concrete shoulder).

References: 8 ft S of A SINGLE LIGHTPOST, 9.0 ft NE OF A WITNESS POST, 16.6 ft SW of FOGLINE of I440 Eastbound, 31.7 ft NW of A SINGLE LIGHT POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-53

Latitude(N): 36°06'56.852023"	             Longitude(W): 86°47'44.957314"

D.A. Northing:                649822.7668 (Ft)

D.A. Easting:                 1733546.9492 (Ft)

Elevation:                              532.983 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-53 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 3.57 miles to the station on the right (in the concrete shoulder).

References: 9.6 ft E of A SINGLE LIGHT POLE, 9.8 ft N of WITNESS POST AT ROCK BLUFF, 20.3 ft S of FOGLINE OF I440 Eastbound, 23.7 ft NW of SINGLE DROP INLET, 33.8 ft E of PULL BOX (TDOT COMM)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-54

Latitude(N): 36°06'57.369847"	             Longitude(W): 86°47'35.746335"

D.A. Northing:              649868.9947 (Ft)

D.A. Easting:               1734303.1225 (Ft)

Elevation:                            526.552 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-54 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 3.7 miles to the station on the right (in the concrete shoulder).

References: 15.3 ft S of FOGLINEOF I440 Eastbound, 15.6 ft N of WINTESS POST AT ROCK BLUFF, 17.4 ft NE OF A SINGLE LIGHT POLE,  42.6 ft WSW of SINGLE DROP INLET.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-55

Latitude(N): 36°06'57.416830"	             Longitude(W): 86°47'19.859590"

D.A. Northing:                  649863.2097 (Ft)

D.A. Easting:                  1735606.6455 (Ft)

Elevation:                           527.305 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 3.95 miles to the station on the right (in the concrete shoulder).

References: 4.1 ft N of BASE OF CONCRETE BARRIER, 7.3 ft S of FOGLINE OF I440-Eastbound, 7.4 ft N of WITNESS POST AT ROCK WEST, 54.9 ft E of MILE MARKER #4.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-56

Latitude(N): 36°06'56.966375"N	             Longitude(W): 86°47'05.688590"

D.A. Northing:                649808.3099 (Ft)

D.A. Easting:                 1736768.9906 (Ft)

Elevation:                          530.659 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 4.15 miles to the station on the right (in the concrete shoulder).

References: 4.5 ft. N of the BASE OF THE CONCRETE BARRIER, .5 ft. S of the FOGLINE of I440 Eastbound, 7.4 ft N of WITNESS POST AT THE CONCRETE WALL

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-57

Latitude(N): 36°06'53.303237"	             Longitude(W): 86°46'33.764047"

D.A. Northing:               649416.9948 (Ft)

D.A. Easting:               1739385.4223 (Ft)

Elevation:                        511.945 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-57 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 4.3 miles to the I-65 on the ramp to the right. Follow on the ramp for 0.3 miles to I-65 N and I-65 S split. Station is +/-100 from split on left.

References: 0.7 ft SSW of WITNESS POST , 26.7 ft NNE of FOGLINE OF I-440 Eastbound on RAMP TO I-65 S, 31 ft SSW of FOGLINE OF I-440 Eastbound on RAMP TO I-65 N, 45.6 ft WNW of CENTER OF TWO SINGLE DROP INLETS

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-58

Latitude(N): 36°06'48.795104"	             Longitude(W): 86°46'25.867483"

D.A. Northing:                648955.9907 (Ft)

D.A. Easting:                 1740029.7293 (Ft)

Elevation:                           517.604 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-58 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 4.3 miles to the I-65 on the ramp to the right. Follow on the ramp for 0.3 miles to I-65 N and I-65 S split. Follow I65  S on ramp for 0.22 mils to station on left.

References: 3.4 ft SW of WITNESS POST , 26.5 ft NE of THE FOGLINE OF I440 Eastbound ON RAMP TO I65 S, 28.6 ft SW of THE FOGLINE OF I440 Eastbound ON RAMP TO I65 S, 141 ft SE of SINGLE LIGHT POST WITH 8 LIGHTS.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-59

Latitude(N): 36°06'56.209225"		             Longitude(W): 86°46'29.957265"

D.A. Northing:                   649708.3805 (Ft)

D.A. Easting:                   1739700.0951 (Ft)

Elevation:                             557.555 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-59 driven flush

Route Description:  From the junction of I-440 and I-40, west on Nashville, go east on I-440 for 4.7 miles to the station on the right before the bridge over I65 driven off the concrete shoulder.

References: 4.9 ft SE of THE WITNESS POST, 4.9 ft S BEHIND THE GUARDRAIL, 12.7 ft W of THE Southwest  CORNER OF THE CONCRETE BRIDGE, 18 ft s OF FOGLINE OF I440 Eastbound, 33.8 ft NNW of TRAFFIC CAMERA POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-60

Latitude(N): 36°06'43.940533"	             Longitude(W): 86°46'17.786352"

D.A. Northing:                648459.8490 (Ft)

D.A. Easting:                 1740688.9250 (Ft)

Elevation:                          510.919 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-60 driven flush

Route Description:  From  the junction of I65 N and SR 355 (Exit 78) South of Nashville, Go north on I65 for 2.15 mile to I440 ramp and station on the right.

References: 6.1 ft E of WITNESS POST AT EXIT 80 SIGN, 11.4 ft SW of FOGLINE OF I65 Northbound  OFF RAMP TO I440, 22.1 ft NE OF FOGLINE OF I65 N.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-61

Latitude(N): 36°06'53.769001"	             Longitude(W): 86°46'20.264764"

D.A. Northing:               649455.3322 (Ft)

D.A. Easting:                1740493.4080 (Ft)

Elevation:                          527.739 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-61 driven flush

Route Description:  From the junction of I65 and SR 355 South of Nashville, Go North on I65 for 2.15 miles to on ramp to I440. Follow on ramp to I440 West for 0.20 miles to station on right.

References: 4.1 ft ENE of BEHIND GUARDRAIL, 4.2 ft NE of WITNESS POST AT THE GUARDRAIL POST, 20.3 ft ENE of FOGLINE OF I65 N ON RAMP TO I440W, 165 ft SE of NORTHEAST CORNER OF CONCRETE BRIDGE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-62

Latitude(N): 36°06'57.039406"	             Longitude(W): 86°46'15.789285"

D.A. Northing:                    649783.1483 (Ft)

D.A. Easting:                     1740863.2255 (Ft)

Elevation:                              527.472 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-62 driven flush

Route Description:  From the junction of  I65 AND SR 355 (Exit 8) South of Nashville, Go North on I65 for 2.15 miles to on ramp to I440. Follow on ramp to I440 East for 0.25 miles to station on the left.

References: 1.2 ft SW of WITNESS POST, 12.6 ft NW of FOGLINE OF I65 Northbound  OFF RAMP TO I440E, 27.4 ft SSE of  FOGLINE OF I65 Northbound  OFF RAMP TO I440E, 96 ft E of END OF GUARDRAIL.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-63

Latitude(N): 36°07'10.409284"	             Longitude(W): 86°45'58.252852"

D.A. Northing:                651123.8454 (Ft)

D.A. Easting:                 1742312.6536 (Ft)

Elevation:                           525.142 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-63 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 0.45 miles to station on right driven off concrete shoulder.

References: 3 ft  SE OF BEHIND THE COCNRETE BARRIER, 3.1 FT NW OF WINTESS POST AT CONCRETE WALL, 12.1 FT SE OF FOGLINE OF I440 Eastbound, 15.2 FT WNW OF END OF THE CONCRETE WALL, 75.6 FT SW OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-64

Latitude(N): 36°07'13.628441"	             Longitude(W): 86°45'54.347190"

D.A. Northing:                651446.8684 (Ft)

D.A. Easting:                1742635.6386 (Ft)

Elevation:                          521.901 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-64 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 0.55 miles to station on right driven off concrete shoulder.

References: 1.1 FT SSE OF BEHIND THE GUARDRAIL, 3 FT NNW OF WINTESS POST, 14.3 FT SSE OF FOGLINE OF I440 Eastbound, 105.3 FT WSW OF SINGLE LIGHT POLE #HD21, 133 FT NE OF SINGLE LIGHT POST #HD20.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-65

Latitude(N): 36°07'16.524095"	             Longitude(W): 86°45'48.605054"

D.A. Northing:              651736.5808 (Ft)

D.A. Easting:               1743108.6546 (Ft)

Elevation:                         516.027 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-65 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 0.65 miles to station on right driven off concrete shoulder.

References: 4.9 FT S BEHIND THE GUARDRAIL, 3.4 FT SW OF THE WITNESS POST AT THE GUARDRAIL POST, 15.7 FT S OF THE FOGLINE OF I440 Eastbound, 15.8 FT NE OF THE I440 EAST SIGN, 16.6 FT WSW OF SINGLE LIGT POST  #HD23.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-66

Latitude(N): 36°07'17.611511"	             Longitude(W): 86°45'44.650615"

D.A. Northing:                651843.4388 (Ft)

D.A. Easting:                 1743434.3229 (Ft)

Elevation:                           519.335 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-66 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 0.72 miles to station on right driven off concrete shoulder.

References: 0.9 FT N OF THE FRONT OF THE GUARDRAIL, 3.7 FT NW OF WINTESS POST AT THE GUARDRAIL POST, 12.6 FT S OF FOGLINE OF I440 Eastbound, 29 FT N OF CHAINLINK FENCE, 83.7 FT E OF SINGLE LIGHT POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-67

Latitude(N): 36°07'18.034037"	             Longitude(W): 86°45'40.752653"

D.A. Northing:                   651883.6763 (Ft)

D.A. Easting:                    1743754.4556 (Ft)

Elevation:                             527.783 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-67 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 0.79 miles to station on right driven off concrete shoulder.

References: 13.2 FT S OF FOGLINE OF I440 Eastbound, 16.1 FT N OF WITNESS POST, 71.4 FT W OF SINGLE LIGHT POLE #HD26, 102 FT W OF SPEED LIMIT 55 SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-68

Latitude(N): 36°07'17.922733"	             Longitude(W): 86°45'37.135704"

D.A. Northing:                   651870.1140 (Ft)

D.A. Easting:                    1744051.1130 (Ft)

Elevation:                             536.727 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-68 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 0.82 miles to station on right driven off concrete shoulder.

References:  10.5 FT N OF WITNESS POST, 14.7 FT S OF FOGLINE OF I440 Eastbound,  15 FT NW OF SINGLE LIGHT POLE #HD27

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-69

Latitude(N): 36°07'16.739914"	             Longitude(W): 86°45'24.377860"

D.A. Northing:                  651742.3913 (Ft)

D.A. Easting:                   1745096.8783 (Ft)

Elevation:                             567.718 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-69 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 1.65  miles to station on right driven off concrete shoulder.

References: 12.3 FT N OF WITNESS POST AT ROCK BLUFF, 15.8 FT S OF FOGLINE OF I440 Eastbound, 35.6 FT E OF SINGLE LIGHT POLE, 108 FT E OF AIRPORT INFO SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-70

Latitude(N): 36°07'18.234256"	             Longitude(W): 86°45'14.916828"

D.A. Northing:                 651887.5115 (Ft)

D.A. Easting:                  1745874.2566 (Ft)

Elevation:                           567.471 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-70 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 1.18 miles to station on right driven off concrete shoulder at Exit 6 off ramp.

References: 1.7 FT NNW OF WITNESS POST, 13.1 FT NNWOF FOGLINE OF I440 Eastbound OFF RAMP TO SR11 (EXIT 6 NOLENSVILLE RD) , 20 FT SSE OF FOGLINE OF I440 Eastbound, 39.5 WSW OF EXIT 6 SIGN, 102 FT WSW OF SINLE LIGHT POLE

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-71

Latitude(N): 36°07'20.326899"	             Longitude(W): 86°45'08.506014"

D.A. Northing:               652095.0774 (Ft)

D.A. Easting:                1746401.8447 (Ft)

Elevation:                          556.704 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-71 driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 1.3  miles to station on right driven off concrete shoulder at SR 11 on ramp to 440E.

References: 13.6 FT SSE OF FOGLINE OF I440 Eastbound, 18 FT NNE OF WITNESS POST AT THE SINGLE LIGHT POLE, 19 FT N OF SINGLE DROP INLET, 21 FT NW OF EXIT SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-72

Latitude(N): 36°07'23.417180"	             Longitude(W): 86°44'58.569524"

D.A. Northing:                     652401.3199 (Ft)

D.A. Easting:                      1747219.4503 (Ft)

Elevation:                               532.548 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 1.42  miles to station on right driven off concrete shoulder.

References: 2.9 NW OF BASE FO CONCRETE BARRIER, 5.1 FT SE OF FOGLINE OF I440 Eastbound,  48.2 FT OF SINGLE DROP INLET

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-73

Latitude(N): 36°07'24.732332"	             Longitude(W): 86°44'52.014769"

D.A. Northing:                     652530.1980 (Ft)

D.A. Easting:                      1747758.2275 (Ft)

Elevation:                               527.761 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 0.45 miles to station on right driven off concrete shoulder.

References: 3.2 FT S OF FOGLINE OF I440 Eastbound, 4.8 FT N OF BASE OF CONCRETE BARRIER, 72 FT W OF SINGLE DROP INLET, 79 FT W OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-74

Latitude(N): 36°07'25.805373"	             Longitude(W): 86°44'44.973235"

D.A. Northing:                 652634.2988 (Ft)

D.A. Easting:                  1748336.7490 (Ft)

Elevation:                           524.334 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 1.67 miles to station on right driven off concrete shoulder.

References: 4 FT S OF FOGLINE OF I440 Eastbound,4.2 FT N OF BASE,  32.6 FT W OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-75

Latitude(N): 36°07'26.338503"	             Longitude(W): 86°44'39.815737"

D.A. Northing:                   652684.9889 (Ft)

D.A. Easting:                   1748760.2839 (Ft)

Elevation:                             521.662 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 1.75 miles to station on right driven off concrete shoulder.

References: 3.7 FT N OF BASE OF CONCRETE BARRIER, 4.5FT S OF FOGLINE OF I440 Eastbound, 57.5 FT W OF SINGLE DROP INLET, 90.6 FT W OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-76

Latitude(N): 36°07'27.250262"	             Longitude(W): 86°44'19.406107"

D.A. Northing:                 652764.5013 (Ft)

D.A. Easting:                 1750435.3976 (Ft)

Elevation:                             522.564 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 2.05 miles to station on right driven off concrete shoulder.

References:  4.9 FT N OF BASE OF CONCRETE BARRIER, 7.3 FT S OF FOGLINE OF I440 Eastbound, 8.8FT N WITNESS POST AT CONCRETE WALL, 65.4 FT WNW OF END OF CONCRETE BRIDGE

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-77

Latitude(N): 36°07'28.517167"	             Longitude(W): 86°44'08.506538"

D.A. Northing:                652885.8786 (Ft)

D.A. Easting:                 1751330.5652 (Ft)

Elevation:                           531.706 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I65 AND I440 South of Nashville, Go East on I440 for 2.25 miles to station on right driven off concrete shoulder.

References: 3.7 FT NOF BASE OF CONCRETE BARRIER, 6.7 FT S OF FOGLINE OF I440 Eastbound, 9.1 FT N WITNESS POST AT CONCRETE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-78

Latitude(N): 36°09'20.043897"	             Longitude(W): 86°49'45.514075"

D.A. Northing:                 664384.8257 (Ft)

D.A. Easting:                  1723778.1419 (Ft)

Elevation:                            575.505 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped19-440-78 driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .55 mile to station on right.

References: 4.2 FT W OF WITNESS POST, 12.6 FT N OF FOGLINE I40 West, 55.5 W  SINGLE LIGHT POLE #DM17

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-79

Latitude(N): 36°09'23.287697"	             Longitude(W): 86°49'37.039160"

D.A. Northing:                    664706.9679 (Ft)

D.A. Easting:                     1724475.9190 (Ft)

Elevation:                                  600.431 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-79 driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .4  mile to station on right.

References: 3.4 FT E OF WITNESS POST, 13.3 FT N OF FOGLINE I40 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-80

Latitude(N): 36°09'24.455476"	             Longitude(W): 86°49'32.033013"

D.A. Northing:                       664821.5912 (Ft)

D.A. Easting:                        1724887.4516 (Ft)

Elevation:                                 613.521 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-80 driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .32  mile to station on right.

References: 3.2 ft E OF WITNESS POST, 9.0 FT W OF END OF BRIDGE WALL, 15.0 FT N OF FOGLINE I40 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-81

Latitude(N): 36°09'21.820976"	             Longitude(W): 86°49'24.005300"

D.A. Northing:                   664549.6364 (Ft)

D.A. Easting:                    1725543.5319 (Ft)

Elevation:                              602.477 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-81 driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .2 mile to station on right.

References: 3.1 FT W OF WITNESS POST, 13.9 FT E OF FOGLINE I40 West ON RAMP

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-82

Latitude(N): 36°09'24.081644"	             Longitude(W): 86°49'23.681707"

D.A. Northing:                    664778.0191 (Ft)

D.A. Easting:                    1725571.9918 (Ft)

Elevation:                              612.553 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .15 mile to station on right.

References: 4.5 FT SE OF WITNESS POST, 8.7 FT NW OF CENTER OF FOGLINE I40 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-83

Latitude(N): 36°09'22.820487"	             Longitude(W): 86°49'23.713264"

D.A. Northing:                664650.5086 (Ft)

D.A. Easting:                  1725568.3311(Ft)

Elevation:                           618.419 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .17 mile to station on right.

References: 4.7 FT OF WITNESS POST, 11.3 FT OF NW OF CENTER OF FOGLINE I40 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-84

Latitude(N): 36°09'28.626376"	             Longitude(W): 86°49'20.161256"

D.A. Northing:                             665235.1640 (Ft)

D.A. Easting:                              1725864.5760 (Ft)

Elevation:                                      571.597 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-84 driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .25 mile to station on left.

References: 1.9 FT SE OF WITNESS , 12.3 FT NW OF FOGLINE ON I40 East, 26.5 FT SE OF FOGLINE I40 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-85

Latitude(N): 36°09'23.770659"	             Longitude(W): 86°49'20.657456"

D.A. Northing:                   664744.4803 (Ft)

D.A. Easting:                    1725819.7103 (Ft)

Elevation:                              584.059 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-85 driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go east on I40 for .55 mile to station on right.

References: 2.7 FT W OF WITNESS POST, 7.1 FT S OF END OF THE GUARDRAIL, 13.3 FT E OF FOGLINE ON I40 East, 36.1 FT SW SINGLE LIGHT POLE # DL44.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-86

Latitude(N): 36°09'18.226715"	             Longitude(W): 86°49'21.393778"

D.A. Northing:                  664184.3724 (Ft)

D.A. Easting:                   1725754.6397 (Ft)

Elevation:                             589.409 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-86 driven flush

Route Description:  From the junction of  I440 west  to I40 East/West split . Go west on I40 for .1 mile to station on right.

References: 2.1 FT W OF WITNESS POST, 15.4 FT W OF FOGLINE I40 West, 20.2 FT E OF FOGLINE I40 East ON RAMP.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-87

Latitude(N): 36°09'13.481240"	             Longitude(W): 86°49'20.693709"

D.A. Northing:               663704.0126 (Ft)

D.A. Easting:                1725808.0182 (Ft)

Elevation:                          596.017 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-87 driven flush

Route Description:  From the junction of  I440 west  and I65. Go westerly on I440 for 4.55 miles to station on the right.

References: 6.4 FT N OF WITNESS POST AT GUARDRAIL, 13.4 FT E OF FOGLINE ON I440 West, 33.0 FT W OF FENCE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-88

Latitude(N): 36°09'08.543633"	             Longitude(W): 86°49'20.717914”

D.A. Northing:                    663204.7233 (Ft)

D.A. Easting:                     1725801.8371 (Ft)

Elevation:                                  611.884 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440 west  and I65 . Go westerly on I440 for 4.55 mile to station on right.

References: 7.4 FT E OF FOGLINE ON I440 West, 12.9 FT W OF FENCE, 55.9 FT SW OF SINGLE LIGHT POST #HM04.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-89

Latitude(N): 36°09'01.778215"	             Longitude(W): 86°49'20.575312"

D.A. Northing:                    662520.4852 (Ft)

D.A. Easting:                     1725807.7832 (Ft)

Elevation:                                 607.851 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 4.35 mile to station on right.

References: 9.6 FT W OF SINGLE LIGHT POLE #HM07, 9.8 FT E OF FOGLINE I440 West, 16.3 FT SW OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-90

Latitude(N): 36°08'54.424255"	             Longitude(W): 86°49'19.745182"

D.A. Northing:                661776.2587 (Ft)

D.A. Easting:                 1725869.6170 (Ft)

Elevation:                          594.637 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-90 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 4.2 mile to station on right.

References: 4.7 FT WSW OF WITNESS POST, 5.3 FT WNW OF TDOT CAMERA POLE #CCTV63, 7.6 FT SSW OF SINGLE  LIGHT POLE #HM10, 13.2 FT NW OF Northeast CORNER OF BRIDGE, 14.6 FT ENE FOGLINE I440 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-91

Latitude(N): 36°08'47.034300"	             Longitude(W): 86°49'18.851179"

D.A. Northing:               661028.3488 (Ft)

D.A. Easting:                1725936.6624 (Ft)

Elevation:                          587.451 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-91 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 4.05 mile to station on right.

References: 3.1 FT W OF WITNESS POST, 8.8 FT NW OF SINGLE LIGHT POLE #HM13, 16.0 FT SW OF 10” PINE TREE, 16.3 FT E OF FOGLINE ON I440 West, 17.5 FT W OF FENCE

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-92

Latitude(N): 36°08'40.494839"	             Longitude(W): 86°49'19.291319"

D.A. Northing:               660367.3628 (Ft)

D.A. Easting:                1725895.0090 (Ft)

Elevation:                          567.091 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-92 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 3.9  mile to station on right.

References: 3.4 FT W OF WITNESS POST, 12.0 FT NNW OF SINGLE LIGHT POLE #HM16, 14.7 FT NW OF END OF BLUFF, 15.7 E OF FOGLINE ON I440 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-93

Latitude(N): 36°08'31.211407"	             Longitude(W): 86°49'21.254049"

D.A. Northing:               659429.9841 (Ft)

D.A. Easting:                1725726.1705 (Ft)

Elevation:                          539.344 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-93 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 3.75 mile to station on right.

References: 27.7 FT SSW OF WITNESS POST AT MERGING LANES SIGN, 13 FT ESE OF FOGLINE ON I440 West, 19 FT W OF I440 West ON RAMP.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-94

Latitude(N): 36°08'23.383758"	             Longitude(W): 86°49'18.776588"

D.A. Northing:                  658636.6876 (Ft)

D.A. Easting:                  1725922.6952 (Ft)

Elevation:                            573.368 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-94 driven flush

Route Description:  From the junction of  I440  West  to I65 on I440 for 3.5 miles to Murphy Road. The point is in the N.E corner of interchange at the top of I440 West.

References: 3.3 FT E OF WITNESS POST, 17 FT W OF FOGLINE I440 West ON RAMP, 32.5 FT SINGLE LIGHT POLE #HJ07, 45.3 FT N FOGLIEN MURPHY ROAD.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-95

Latitude(N): 36°08'23.829007"	             Longitude(W): 86°49'21.437049"

D.A. Northing:                    658683.5458 (Ft)

D.A. Easting:                     1725704.8530 (Ft)

Elevation:                              540.058 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-95 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 3.6 mile to station on right.

References: 12 FT WSW OF WITNESS POST AT WING WALL, 18.7 FT ENE OF FOGLINE ON I440 West, 55.8 FT NNW OF EDGE OF ABUTMENT.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-96

Latitude(N): 36°08'21.286891"	             Longitude(W): 86°49'20.815426"

D.A. Northing:                 658426.0516 (Ft)

D.A. Easting:                  1725753.6803 (Ft)

Elevation:                           541.157 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-96 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 3.55 mile to station on right.

References: 12 FT WSW OF WITNESS POST AT BLUFF, 18.6 FT ENE OF FOGLINE ON I440 West, 28.5 FT SE OF SINGLE DROP INLET.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-97

Latitude(N): 36°08'16.688001"	             Longitude(W): 86°49'18.777941"

D.A. Northing:                 657959.5954 (Ft)

D.A. Easting:                  1725916.8980 (Ft)

Elevation:                            544.880 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-97 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 3.45 mile to station on right.

References: 11.0 FT SE PF WITNESS POST AT MERGING LANES SIGN, 11.4 FT SW OF I440 West ON RAMP, 20.8 FT ENE OF FOGLINE ON I440 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-98

Latitude(N): 36°08'05.662950"	             Longitude(W): 86°49'11.469429"

D.A. Northing:                 656839.6844 (Ft)

D.A. Easting:                  1726507.0433 (Ft)

Elevation:                           552.805 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-98 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 3.2 mile to station on right.

References: 11.7 FT SW OF I440 West ON RAMP FOGLINE, 14.8 FT NW OF WITNESS POST AT EXIT 1 SIGN, 27.5 FT NE OF FOGLINE ON I440 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-99

Latitude(N): 36°07'56.323094"	             Longitude(W): 86°49'05.332087"

D.A. Northing:                 655891.0007 (Ft)

D.A. Easting:                 1727002.5863 (Ft)

Elevation:                              556.800 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-99 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 3.0 miles to station on right.

References: 14.0 FT WSW OF WITNESS POST AT ROCK BLUFF, 16.5 FT ENE OF FOGLINE ON I440 West, 32.4 FT S OF SINGLE LIGHT POLE #HI33.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-100

Latitude(N): 36°07'45.868990"	             Longitude(W): 86°48'58.483126"

D.A. Northing:                 654829.1412 (Ft)

D.A. Easting:                  1727555.5569 (Ft)

Elevation:                           549.600 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-100 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 2.8  mile to station on right.

References: 12.5 FT WSW OF WITNESS POST AT ROCK BLUFF, 16.2 FT ENE OF FOGLINE ON I440 West, 30.7 FT NW OF 55MPH SIGN, 47.2 FT SSW OF SINGLE LIGHT POLE #HI02.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-101

Latitude(N): 36°07'39.601404"	             Longitude(W): 86°48'54.396143"

D.A. Northing:                   654192.5665 (Ft)

D.A. Easting:                   1727885.6287 (Ft)

Elevation:                             544.751 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-101 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 2.65  mile to station on right.

References: 13 FT S OF SINGLE LIGHT POLE #HI05, 15 FT ENE OF FOGLINE I440 West, 36.5 FT WSW OF WITNESS POST AT THE SOUND WALL, 47.7 FT NW OF THE END OF THE CONCRETE BARRIER.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-102

Latitude(N): 36°07'34.813884"	             Longitude(W): 86°48'51.340382"

D.A. Northing:                653706.3623 (Ft)

D.A. Easting:                 1728132.2495 (Ft)

Elevation:                           541.630 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 2.5 mile to station on right.

References: 9 FT NE OF FOGLINE ON I440 West, 27 FT SW OF WITNESS POST AT THE SOUNDWALL, 57.5 FT SSE OF SINGLE LIGHT POLE #HI09, 62 FT NW OF EDGE OF CONCRETE ABUTMENT, 76 FT SSE OF SINGLE DROP INLET.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-103

Latitude(N): 36°07'31.057822"	             Longitude(W): 86°48'48.076323"

D.A. Northing:                    653324.3135 (Ft)

D.A. Easting:                     1728396.8686 (Ft)

Elevation:                              550.328 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-103 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 2.45 mile to station on right.

References: 13.5 FT SW OF WITNESS POST AT ROCK BLUFF, 15.6 FT NE OF FOGLINE ON I440 West, 42.8 FT SSW OF SINGLE DROP INLET.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-104

Latitude(N): 36°07'28.679590"	             Longitude(W): 86°48'44.556060"

D.A. Northing:                     653081.4221 (Ft)

D.A. Easting:                      1728683.6703 (Ft)

Elevation:                                   560.232 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-104 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 2.4  mile to station on right.

References: 13.7 FT ENE OF FOGLINE ON I440 West, 16 FT WSW OF WITNESS POST AT ROCK BLUFF, 26 FT NNW OF SINGLE LIGHT POLE #HI10, 73.5 FT NNW OF SINGLE LIGHT POLE, 77 FT SSE OF END OF GUARDRAIL.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-105

Latitude(N): 36°07'27.300905"	             Longitude(W): 86°48'40.387694"

D.A. Northing:                   652939.1695 (Ft)

D.A. Easting:                    1729024.4850 (Ft)

Elevation:                              566.842 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped  19-440-105 driven flush

Route Description:  From the junction of I440 West  to I65 . Go westerly on I440 for 2.3 mile to station on right.

References: 13.3 FT ENE OF FOGLINE ON I440 West, 14.7 FT WSW OF WITNESS POST AT ROCK BLUFF, 32.4 FT NW OF SINGLE LIGHT POLE #HH19

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-106

Latitude(N): 36°07'26.002625"	             Longitude(W): 86°48'34.778387"

D.A. Northing:                   652804.0734 (Ft)

D.A. Easting:                    1729483.5856 (Ft)

Elevation:                              570.218 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-106 driven flush

Route Description:  From the junction of I440 West to I65. Go westerly on I440 for 2.2 mile to station on right.

References: 10 FT ESE OF WITNESS POST AT MERGING LANES SIGN, 15.4 FT SSW OF FOGLINE I440 West ON RAMP, 18.5 FT NNE OF FOGLINE ON I440 West, 49.6 FT WNW OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-107

Latitude(N): 36°07'24.331503"	             Longitude(W): 86°48'26.159196"

D.A. Northing:               652629.2440 (Ft)

D.A. Easting:                1730189.3120 (Ft)

Elevation:                              561.457 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-107 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 2.05  mile to station on right.

References: 8.6 FT S OF WITNESS POST, 10.0 FT W OF FOGLINE OF EXIT RAMP, 14.0 FT N OF FOGLINE ON I440 West, 81.9 FT E OF SINGLE DROP INLET.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-108

Latitude(N): 36°07'03.396210"	             Longitude(W): 86°47'59.809189"

D.A. Northing:                    650494.4651 (Ft)

D.A. Easting:                     1732333.7818 (Ft)

Elevation:                               561.482 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-108 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 1.5  mile to station on right.

References: 12.7 FT S OF WITNESS POST, 19.1 FT N OF FOGLINE ON I440 West, 119.6 FT W OF SINGLE LIGHT POLE #HF19, 120.7 FT E OF SINGLE LIGHT POLE #HF20.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-109

Latitude(N): 36°07'01.287288"	             Longitude(W): 86°47'57.263685"

D.A. Northing:                  650279.4801 (Ft)

D.A. Easting:                   1732540.7671 (Ft)

Elevation:                                 547.667 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-109 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 1.45  mile to station on right.

References: 14.8 FT N OF FOGLINE ON I440 West, 15.5 FT S OF WITNESS POST, 48.0 FT E OF SINGLE DROP INLET.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-110

Latitude(N): 36°06'59.750654"	             Longitude(W): 86°47'54.251631”

D.A. Northing:                 650122.0945 (Ft)

D.A. Easting:                  1732786.7541 (Ft)

Elevation:                               537.835 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-110 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 1.4 mile to station on right.

References: 6.2 FT SW OF SINGLE LIGHT POLE, 15.4 FT S OF WITNESS POST, 15.9 FT N OF FOGLINE ON I440 West, 34.8 FT E OF SINGLE DROP INLET.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-111

Latitude(N): 36°06'58.640451"	             Longitude(W): 86°47'50.278324"

D.A. Northing:                  650007.1704 (Ft)

D.A. Easting:                   1733111.8408 (Ft)

Elevation:                            533.017 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-111 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 1.35  mile to station on right.

References: 14.3 FT S PF WITNESS POST, 16.1 FT N OF FOGLINE ON I440 West, 68.0 FT W OF SINGLE LIGHT POLE #HF15

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-112

Latitude(N): 36°06'58.345456"	             Longitude(W): 86°47'46.944971"

D.A. Northing:               649975.1130 (Ft)

D.A. Easting:                1733385.0935 (Ft)

Elevation:                             531.295 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-112 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 1.25  mile to station on right.

References: 14.6 FT S OF WITNESS POST AT ROCK BLUFF, 15.6 FT N OF FOGLINE ON I440 West, 22.7 FT W OF SINGLE  LIGHT POLE #HF14.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-113

Latitude(N): 36°06'58.842508"	             Longitude(W): 86°47'36.106520"

D.A. Northing:                    650018.1633 (Ft)

D.A. Easting:                    1734274.7551 (Ft)

Elevation:                              528.030 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-113 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 1.1  mile to station on right.

References: 12.5 FT E  OF SINGLE LIGHT POLE #HF11, 13.1 FT S PF WITNESS POST AT ROCK BLUFF, 17.0 FT N OF FOGLINE ON I440 West, 30.4 FT W OF 55 MPH SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-114

Latitude(N): 36°06'58.877682"	             Longitude(W): 86°47'22.123489"

D.A. Northing:                650012.4323 (Ft)

D.A. Easting:                 1735422.0875 (Ft)

Elevation:                          527.138 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-114 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 0.9  mile to station on right.

References: 14.1 FT S OF WITNESS POST AT ROCK BLUFF, 15.5 FT N OF FOGLINE ON I440 West, 36.6 FT E OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-115

Latitude(N): 36°06'58.535022"	             Longitude(W): 86°47'14.097442"

D.A. Northing:              649972.4770 (Ft)

D.A. Easting:              1736080.3311 (Ft)

Elevation:                            529.761 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 0.75 mile to station on right.

References: 6.2 FT N OF FOGLINE ON I440 West, 7.2 FT S  OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-116

Latitude(N): 36°06'58.172373"	             Longitude(W): 86°47'02.401586"

D.A. Northing:                649928.1015 (Ft)

D.A. Easting:                1737039.6620 (Ft)

Elevation:                         527.556  (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 0.55  mile to station on right.

References: 5.6 FT S OF WITNESS POST, 8.0 FT N OF FOGLINE ON I440 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-117

Latitude(N): 36°06'57.628397"	             Longitude(W): 86°46'46.962236"

D.A. Northing:                  649862.9730 (Ft)

D.A. Easting:                   1738305.9977 (Ft)

Elevation:                            508.532 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-117 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 0.3  mile to station on right.

References: 14.6 FT N OF FOGLINE ON I440 West, 17.0 FT S OF FOGLINE ON I440 West ON RAMP, 79.0 FT W OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-118

Latitude(N): 36°06'59.499316"	             Longitude(W): 86°46'34.615840"

D.A. Northing:                650044.1120 (Ft)

D.A. Easting:                1739320.5028 (Ft)

Elevation:                          499.230 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-118 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 0.45  mile to station on right.

References: 9.4 FT SE OF FOGLINE ON I440 RAMP, 18.8 NW OF FOGLINE ON I440 RAMP, 38.6 FT W OF WITNESS POST

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-119

Latitude(N): 36°07'02.684899"	             Longitude(W): 86°46'29.566282"

D.A. Northing:              650362.9629 (Ft)

D.A. Easting:               1739737.3598 (Ft)

Elevation:                        503.089 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-119 driven flush

Route Description:  From the junction of  I440  West  to I65 . Go westerly on I440 for 0.3  mile to station on right.

References: 2.4 FT E OF WITNESS POST, 29.5 FT W OF FOGLINE ON I440 West RAMP, 69.2 FT NE OF FOGLINE ON I440 East RAMP.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-120

Latitude(N): 36°07'09.519516"	             Longitude(W): 86°46'27.369075"

D.A. Northing:            651052.6703 (Ft)

D.A. Easting:             1739923.1022 (Ft)

Elevation:                        503.474 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I440  West  to I65 S . Go westerly on I440 for .15 mile to station on right.

References: 5.7 FT S OF GUARDRAIL, 6.7 FT N OF FOGLINE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-121

Latitude(N): 36°06'58.709830"	             Longitude(W): 86°46'27.805883"

D.A. Northing:               649959.8507 (Ft)

D.A. Easting:                1739878.6148 (Ft)

Elevation:                           503.599 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-121 driven flush

Route Description:  From the junction of  I440  West  to I65 N . Go northerly on I65 for 0.3  mile to station on right.

References: 5.2 ft N OF GUARDRAIL, 21.0 FT N OF FOGLINE, 21.2 FT SW OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-122

Latitude(N): 36°06'57.332273"	             Longitude(W): 86°46'32.064265"

D.A. Northing:               649823.3154 (Ft)

D.A. Easting:                1739528.1179 (Ft)

Elevation:                           548.716 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-122 driven flush

Route Description:  From the junction of I65 S and I440 West split, go westerly on I440 for 2.6 miles to station on right.

References: 4.7  FT W OF WITNESS POST AT THE GUARDRAIL, 15.5 FT N OF FOGLINE ON I440 West ON RAMP.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-123

Latitude(N): 36°07'00.731970"	             Longitude(W): 86°46'15.286594"

D.A. Northing:                 650156.2249 (Ft)

D.A. Easting:                  1740907.4120 (Ft)

Elevation:                            540.311 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-123 driven flush

Route Description:  From the junction of I65 S and I440 West split, go westerly on I440 for 0.1 miles to station on right.

References: 3.1 FT S OF WITNESS POST , 15.7 FT N OF FOGLINE ON I440 West ON RAMP, 70.6 FT E OF END OF GUARDRAIL, 100.7 FT W OF 55 MPH SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-124

Latitude(N): 36°07'08.398102"	             Longitude(W): 86°46'21.743410"

D.A. Northing:                   650935.6221 (Ft)

D.A. Easting:                   1740383.7658 (Ft)

Elevation:                            507.742 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of I65 S/N split and I440 West , go southerly on I65 for .35  miles to station on right.

References: 3.5 FT W  OF CONCRETE WALL, 5.6 FT E OF FOGLINE ON I65 North ON RAMP, 104 FT S CENTER OF BRIDGE OVER BERRY ROAD.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-125

Latitude(N): 36°07'06.665227"	             Longitude(W): 86°46'21.066143"

D.A. Northing:               650759.9506 (Ft)

D.A. Easting:                1740437.9492 (Ft)

Elevation:                             514.707 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-125 driven flush

Route Description:  From the junction of I65 S/N split and I440 West , go southerly on I65 for .3  miles to station on right.

References: 7.4 FT SW OF FOGLINE ON I65 North ON RAMP, 11.7 FT NW OF WITNESS POST AT THE END OF THE CONCRETE WALL, 180 FT SSE OF MERGING LANES SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-126

Latitude(N): 36°07'04.207822"	             Longitude(W): 86°46'17.545998"

D.A. Northing:                    650509.1730 (Ft)

D.A. Easting:                    1740724.8041 (Ft)

Elevation:                            517.646 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-126 driven flush

Route Description:  From the junction of I65 S/N split and I440 West , go southerly on I65 for .25  miles to station on right.

References: 6.4 FT SW OF FOGLINE ON I65 North ON RAMP, 8.3 FT SE OF WITNESS POST AT THE END OF THE GUARDRAIL, 118 FT NNE OF SINGLE LIGHT POLE WITH 8 LIGHTS

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-127

Latitude(N): 36°07'03.363221"	             Longitude(W): 86°46'12.846467"

D.A. Northing:                       650420.7275 (Ft)

D.A. Easting:                        1741109.7131 (Ft)

Elevation:                                513.275 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-127 driven flush

Route Description:  From the junction of I65 S/N split and I440 West , go southerly on I65 for .15  miles to station on right.

References: 11 FT E OF WITNESS POST, 17 FT S OF FOGLINE ON I65 South ON RAMP, 25 FT N OF FOGLINE I65 South ON RAMP, 114.5 FT E OF 45 MPH/LEFT TURN SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-128

Latitude(N): 36°07'05.599794"	             Longitude(W): 86°46'07.432559"

D.A. Northing:                650643.4031 (Ft)

D.A. Easting:                1741555.6845 (Ft)

Elevation:                         511.989 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-128 driven flush

Route Description:  From the junction of I65 S/N split and I440 West , go southerly on I65 for .07  miles to station on right.

References: 2.7 FT SE OF WITNESS POST, 14 FTNW OF FOGLINE ON I65 South ON RAMP, 51 FT NE OF SINGLE LIGHT POLE #HD10, 102 FT SW OF SIGNLE LIGHT POLE #HD09.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-129

Latitude(N): 36°07'08.363315"	             Longitude(W): 86°46'03.260148"

D.A. Northing:              650920.2016 (Ft)

D.A. Easting:               1741900.2063 (Ft)

Elevation:                         518.546 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-129 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 2.5 miles to station on right.

References: 10.7 FT N OF  FOGLINE ON I440 West, 11.4 FT S OF CONCRETE WALL, 12.2 FT W OF WITNESS POST, 17.3 FT W OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-130

Latitude(N): 36°07'10.912993"	             Longitude(W): 86°46'00.090419"

D.A. Northing:                 651175.9624 (Ft)

D.A. Easting:                  1742162.2896 (Ft)

Elevation:                           522.642 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 2.4 miles to station on right.

References: 4.1 FT N OF FOGLINE ON I440 West, 4.9 FT S OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-131

Latitude(N): 36°07'15.046438"	             Longitude(W): 86°45'54.854514"

D.A. Northing:                  651590.5854 (Ft)

D.A. Easting:                   1742595.1368 (Ft)

Elevation:                            528.312 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 2.3 miles to station on right.

References: 4.7 FT N OF FOGLINE ON I440 West, 5.3 FT S OF WITNESS POST, 18.3 FT E OF END OF GUARDRAIL, 170.4 FT W OF SINGLE LIGHT POLE #HD45

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-132

Latitude(N): 36°07'18.922618"	             Longitude(W): 86°45'45.372630"

D.A. Northing:              651976.4831 (Ft)

D.A. Easting:              1743376.1199 (Ft)

Elevation:                        525.883 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-132 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 2.15 miles to station on right.

References: 10.1 FT E OF WITNESS POST AT GUARDRAIL, 12.5 FT S OF CONCRETE DITCH, 12.7 FT N OF FOLGLINE ON I440 West, 67.1 FT W OF SINGLE LIGHT POLE #HD42

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-133

Latitude(N): 36°07'19.440493"	             Longitude(W): 86°45'38.301577"

D.A. Northing:                 652024.2674 (Ft)

D.A. Easting:                  1743956.6217 (Ft)

Elevation:                            540.588 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-133 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 2.05 miles to station on right.

References: 3.2 FT S OF WITNESS POST, 9.8 FT E OF SINGLE LIGHT POLE #HD40, 12.9 FT N OF FOGLINE ON I440 West.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-134

Latitude(N): 36°07'18.327416"	             Longitude(W): 86°45'26.577784"

D.A. Northing:                  651904.3202 (Ft)

D.A. Easting:                   1744917.6319 (Ft)

Elevation:                            558.183 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-134 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.85  miles to station on right.

References: 5.0 FT S OF WITNESS POST, 13.1 FT N OF FOGLINE ON I440 West, 27.3 FT SE OF END OF WALL, 35.2 FT W OF SINGLE LIGHT POLE #HB12

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-135

Latitude(N): 36°07'18.361575"	             Longitude(W): 86°45'22.542038”

D.A. Northing:                    651905.2139 (Ft)

D.A. Easting:                     1745248.7623 (Ft)

Elevation:                              562.067 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped  19-440-135 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.8  miles to station on right.

References: 3.6 FT S OF WITNESS POST, 13.8 FT N OF FOGLINE ON I440 West, 63.0 FT E OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-136

Latitude(N): 36°07'18.969828"	             Longitude(W): 86°45'18.556640"

D.A. Northing:                 651964.2291 (Ft)

D.A. Easting:                 1745576.2352 (Ft)

Elevation:                           563.597 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-136 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.75  miles.

References: 3.0 FT S OF WITNESS POST, 12.9 FT N OF FOGLINE ON I440 West, 20.4 FT W OF SINGLE LIGHT POLE #HC04, 28.8 FT E OF MILE MARKER #6.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-137

Latitude(N): 36°07'21.571084"	             Longitude(W): 86°45'09.852009"

D.A. Northing:              652221.7422 (Ft)

D.A. Easting:              1746292.3840 (Ft)

Elevation:                        558.206 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-137 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.6  miles to station on right.

References: 1.5 FT S OF WITNESS POST, 7.1 FT S OF FOGLINE OF EXIT RAMP, 12.9 FT N OF FOGLINE ON I440 West, 111.0 FT W OF END OF THE CONCRETE WALL

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-138

Latitude(N): ,36°07'24.547523"	             Longitude(W): 86°45'00.551795"

D.A. Northing:              652516.8697 (Ft)

D.A. Easting:              1747057.6982 (Ft)

Elevation:                        538.666 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-138 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.4 miles to station on right.

References: 2.9 FT S OF WITNESS POST, 17.0 FT N OF FOGLINE ON I440 West, 26.7 FT E OF FOGLINE OF EXIT RAMP, 50.1 FT W OF SINLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-139

Latitude(N): 36°07'25.726301"	             Longitude(W): 86°44'55.849764"

D.A. Northing:              652633.1168 (Ft)

D.A. Easting:              1747444.3699 (Ft)

Elevation:                        534.259 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-139 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.35 miles to station on right.

References: 8.0 FT S OF FOGLINE ON EXIT RAMP, 17.0 FT N OF FOGLINE ON I440 West, 32.9 FT E OF WITNESS POST AT EXIT 6 SIGN, 56.9 FT E OF END OF GUARDRAIL.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-140

Latitude(N): 36°07'27.021174"	             Longitude(W): 86°44'47.257536"

D.A. Northing:              652758.6359 (Ft)

D.A. Easting:              1748150.2166 (Ft)

Elevation:                        528.556 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.22  miles to station on right.

References: 6.5 FT S OF WITNESS POST, 9.8 FT N OF FOGLINE ON I440 West, 31.0 FT S OF CONCRETE WALL, 65.6 FT W OF SINGLE LIGHT POLE, 163.2 FT E OF SINGLE LIGHT POLE.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-141

Latitude(N): 36°07'27.899785”	             Longitude(W): 86°44'35.763384"

D.A. Northing:              652840.3437 (Ft)

D.A. Easting:              1749093.9402 (Ft)

Elevation:                        519.673 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Metal survey spike driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 1.05  miles to station on right.

References: 6.6 FT S OF WITNESS POST, 9.2 FT W OF CONCRETE WALL, 9.5 FT N OF FOGLINE ON I440 West, 134.6 E OF 55 MPH SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-142

Latitude(N): 36°07'28.375788”	             Longitude(W): 86°44'21.869273"

D.A. Northing:              652879.8438 (Ft)

D.A. Easting:              1750234.1787 (Ft)

Elevation:                        519.186 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-142 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.8  miles to station on right.

References: 12.2 FT N OF FOGLINE ON I440 West, 21.7 FT S OF WITNESS POST, 29.6 FT W OF END OF GUARDRAIL, 32.6 FT E OF MOVE VEHICLE SIGN.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-143

Latitude(N): 36°07'28.904171"	             Longitude(W): 86°44'14.918834"

D.A. Northing:              652928.9731 (Ft)

D.A. Easting:              1750804.7954 (Ft)

Elevation:                        524.346 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-143 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.7  miles to station on right.

References: 11.9 FT N OF FOGLINE ON I440 West, 15.1 FT S OF WITNESS POST, 27.6 FT E OF EXIT #6 SIGN, 98.2 FT W OF MILE MARKER #7

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-144

Latitude(N): 36°07'29.979797"	             Longitude(W): 86°44'07.664842"

D.A. Northing:              653033.2652 (Ft)

D.A. Easting:              1751400.7294 (Ft)

Elevation:                        529.510 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-144 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.6  miles to station on right.

References: 13.2 FT N OF FOGLINE ONO I440 West, 17.5 FT S OF WITNESS POST, 65.4 FT E OF SINGLE LIGHT POLE #HB04, 95.5 FT W OF SINGLE LIGHT POLE #HB03

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-145

Latitude(N): 36°07'31.837439"	             Longitude(W): 86°44'00.629872"

D.A. Northing:              653216.7844 (Ft)

D.A. Easting:              1751979.2833 (Ft)

Elevation:                        539.496 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-145 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.5 miles to station on right.

References: 13.0 FT N OF FOGLINE ON I440 West, 16.6 FT S  OF WITNESS POST, 80.5 FT E OF SINGLE LIGHT POLE #HB33

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-146

Latitude(N): 36°07'33.790738"	             Longitude(W): 86°43'54.692241"

D.A. Northing:              653410.6612 (Ft)

D.A. Easting:              1752467.8781 (Ft)

Elevation:                        543.755 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped  19-440-146 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.4  miles to station on right.

References: 4.0 FT S PF WITNESS POST, 12.6 FT N OF FOGLINE ON I440 West, 75 FT S OF BRICK WALL.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-147

Latitude(N): 36°07'36.012159"	             Longitude(W): 86°43'49.271474"

D.A. Northing:              653631.9761 (Ft)

D.A. Easting:              1752914.2661 (Ft)

Elevation:                        538.078 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-147 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.3 miles to station on right.

References: 10.1 FT N OF FOGLINE ON I440 West, 22.4 FT S OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-148

Latitude(N): 36°07'39.460767"	             Longitude(W): 86°43'45.056665"

D.A. Northing:              653978.1318 (Ft)

D.A. Easting:              1753262.6384 (Ft)

Elevation:                        523.298 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-148 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.2 miles to station on right.

References: 9.0 FT N OF FOGLINE ON I440 West ON RAMP, 41.0 FT S OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON 		Year: 2017		Point Name: 19-440-149

Latitude(N): 36°07'43.459108"	             Longitude(W): 86°43'42.619693"

D.A. Northing:              654380.9680 (Ft)

D.A. Easting:              1753465.5698 (Ft)

Elevation:                        506.551 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor: 1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-149 driven flush

Route Description:  From the junction of  I24 East and I440 West split go westerly on I440 for 0.15 miles to station on right.

References: 10.3 FT N OF FOGLINE ON I440 West RAMP, 28.8 FT S OF WITNESS POST.

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson 		Year: 2011		Point Name: 19-440-09

Latitude(N): 36°08'59.982880"	             Longitude(W): 86°49'22.235918"

D.A. Northing:            662340.0300 (Ft)

D.A. Easting:             1725670.0631 (Ft)

Elevation:                        606.125 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-09 driven flush

Route Description:  From the junction of I-440 east and I-40 west, just west-southwest of downtown Nashville, go east on I-440 for 0.4 mile to the station on the right.

References: 17.4 W of the FOGLINE of I-440  14.8 ft SW of a DROP INLET  30.7 ft E of an orange WITNESS POST  31.8 ft E of a CONCRETE WALL  47.8 ft S of LIGHT POLE # 25

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose

Geodetic Survey Control Data

County: Davidson 		Year: 2011		Point Name: 19-440-10

Latitude(N): 36°08'16.987328"	             Longitude(W): 86°49'20.78414

D.A. Northing:          657991.2466 (Ft)

D.A. Easting:           1725752.5923 (Ft)

Elevation:                     546.376 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-10 driven flush

Route Description:  From the junction of I-440 east and I-40 west, just west-southwest of downtown Nashville, go east on I-440 for 1.25 mile to the station on the right.

References: 21.0 ft WSW of the FOGLING of I-440  34.0 ENE of an orange WITNESS POST at a guardrail  59.4 ft SE of a DROP INLET  111.4 ft N of a GAS LINE MARKER

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson 		Year: 2011		Point Name: 19-440-11

Latitude(N): 36°08'09.087747"	             Longitude(W): 86°49'15.599978"

D.A. Northing:             657188.8403 (Ft)

D.A. Easting:              1726171.0630 (Ft)

Elevation:                        550.380 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-11driven flush

Route Description:  From the junction of I-440 east and I-40 west, just west-southwest of downtown Nashville, go east on I-440 for 1.4 miles to the station on the right.

References: 15.3 ft SW of the FOGLINE of I-440  16.2 ft NW of an orange WITNESS POST at the end of guardrail  21.7 ft NNW of a SIGN (Traffic Info)  65.5 ft NE of FOGLINE of I-440 entrance ramp

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-12

Latitude(N): 36°07'21.053007"	             Longitude(W): 86°48'20.833273"

D.A. Northing:               652293.9823 (Ft)

D.A. Easting:                1730623.5437 (Ft)

Elevation:                         564.999 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-12 driven flush

Route Description:  From the junction of I-440 and S.R. 106 (U.S. 431 / Hillsboro Rd. / 21st Ave. S.), south-southwest of downtown Nashville, go east on I-440 for 500 feet to the station on the right.

References: 18.7 ft E of an orange WITNESS POST at a right turn sign  23.2 ft SE of LIGHT POLE # D012  31.6 ft SSW of the FOGLINE of I-440  54.7 ft NW of the 2nd RIGHT TURN SIGN

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-13

Latitude(N): 36°07'18.539136"	             Longitude(W): 86°48'16.150575"

D.A. Northing:               652036.6869 (Ft)

D.A. Easting:                1731005.5442 (Ft)

Elevation:                         570.704 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-13 driven flush

Route Description:  From the junction of I-440 and S.R. 106 (U.S. 431 / Hillsboro Rd. / 21st Ave. S.), south-southwest of downtown Nashville, go east on I-440 for 0.1 mile to the station on the right.

References: 1.4 ft NE of an orange WITNESS POST  17.4 ft SW of the FOGLINE of I-440  50.6 ft SE of LIGHT POLE # HG21  55.1 ft E of a FIBER OPTIC CABLE WARNING POST  98.3 ft NW of a SIGN (Right Land Cont.)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-14

Latitude(N): 36°06'57.471875"	             Longitude(W): 86°47'28.751460"

D.A. Northing:         649874.6410 (Ft)

D.A. Easting:          1734877.1464 (Ft)

Elevation:                     526.268 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-14 driven flush

Route Description:  From the junction of I-440 and S.R. 106 (U.S. 431 / Hillsboro Rd. / 21st Ave. S.), south-southwest of downtown Nashville, go east on I-440 for 1.0 mile to the station on the right.

References: 1.8 ft NNW of an orange WITNESS POST  25.1 ft SSE of the FOGLINE of I-440  36.5 ft NE of a DROP INLET  47.8 ft W of a SIGN (Traffic Info)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-15

Latitude(N): 36°06'57.442668"	             Longitude(W): 86°47'23.772270"

D.A. Northing:              649868.3893 (Ft)

D.A. Easting:               1735285.6320 (Ft)

Elevation:                         526.039 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-15 driven flush

Route Description:  From the junction of I-440 and S.R. 106 (U.S. 431 / Hillsboro Rd. / 21st Ave. S.), south-southwest of downtown Nashville, go east on I-440 for 1.1 miles to the station on the right.

References: 15.2 ft NE of a SIGN (511)  16.0 ft W of the end of GAURDRAIL  16.8 ft S of the FOGLINE of I-440  17.3 ft N of an orange WITNESS POST at a concrete wall  31.6 ft E of LIGHT POLE # HE34

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-16

Latitude(N): 36°06'56.347372"	             Longitude(W): 86°46'46.096834"

D.A. Northing:             649732.8643 (Ft)

D.A. Easting:              1738375.9436 (Ft)

Elevation:                         508.721 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-16 driven flush

Route Description:  From the junction of I-440 and S.R. 106 (U.S. 431 / Hillsboro Rd. / 21st Ave. S.), south-southwest of downtown Nashville, go east on I-440 for 1.7 miles to the station on the right.

References: 1.4 ft NNE of an orange WITNESS POST  18.3 ft SSW of the FOGLINE of I-440  31.3 ft NE of LIGHT POLE # HE13  90.9 ft WNW of a 36” FORKED HACKBERRY

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-17

Latitude(N): 36°06'55.880239"	             Longitude(W): 86°46'32.400507"

D.A. Northing:              649676.6927 (Ft)

D.A. Easting:               1739499.3220 (Ft)

Elevation:                         548.002 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-17 driven flush

Route Description:  From the junction of I-440 and S.R. 106 (U.S. 431 / Hillsboro Rd. / 21st Ave. S.), south-southwest of downtown Nashville, go east on I-440 for 1.9 miles to the station on the right.

References: 3.0 ft N of an orange WITNESS POST  36.5 ft S of the FOGLINE of I-440  126.9 ft WSW of the end of GAURDRAIL  147.0 ft ESE of a SIGN (Seed Limit 55)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-19

Latitude(N): 36°07'02.735721"	             Longitude(W): 86°46'08.135232"

D.A. Northing:                        650354.2194 (Ft)

D.A. Easting:                         1741495.7666 (Ft)

Elevation:                                  522.137 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-19 driven flush

Route Description:  From the junction of I-440 and I-65, south of downtown Nashville, go east on I-440 for 0.25 mile to the station on the right.

References: 1.3 ft NNW of an orange WITNESS POST  16.5 ft SSE of the FOGLINE of I-440  38.8 ft NE of a SIGN (Right Lane Cont.)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-22

Latitude(N): 36°07'32.91324"	             Longitude(W): 86°43'49.47782"

D.A. Northing:                  653318.7294 (Ft)

D.A. Easting:                  1752894.9994 (Ft)

Elevation:                            552.405 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-22 driven flush

Route Description:  From the junction of I-440 and I-65, south of downtown Nashville, go east on I-440 for 2.55 miles to I-40 East (Knoxville) and I-24 (Chattanooga) split and the station on the right.

References: 1.7 ft N of an orange WITNESS POST  8.0 ft NE of a TDOT I.T.S.  COMMUNICATION BOX  13.0 ft E of a UTILITY POLE (TDOT I.T.S. RDS-196 197)  26.2 ft S of the FOGLINE of I-440  108.0 ft SW of a SIGN (Left Turn)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-07

Latitude(N): 36°07'33.72508"	             Longitude(W): 86°43'46.68024"

D.A. Northing:                     653399.1134 (Ft)

D.A. Easting:                     1753125.1213 (Ft)

Elevation:                               559.261 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-07 driven flush

Route Description:  From the junction of I-440 and I-65, south of downtown Nashville, go east on I-440 for 2.6 miles to the I-40 East (Knoxville) and I-24 (Chattanooga) split and the station on the right.

References: 2.3 ft SSE of an orange WITNESS POST at a guardrail  13.2 ft SSE of the FOGLINE of I-440  97.4 ft WSW of a SIGN (Murfreesboro Rd. / US 41 / US 70S)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON		Year: 2016		Point Name: 19-440-30

Latitude(N): 36°07'07.153715"	Longitude(W): 86°48'03.260505"

D.A. Northing:                      650876.7490 (Ft)

D.A. Easting:                       1732053.7221 (Ft)

Elevation:                               576.411 (Ft)

Horz. Datum:  NAD83 (1995)	          		D.A. Factor: 1.00006

Vert. Datum:  NAVD 1988	            	Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:  Aluminum disk atop 5/8” rebar stamped 19-440-30 driven flush

Route Description:  From the junction of I-65 and I-440, south of Nashville, go west on I-440 for 1.6 miles to the station on the right.

References:  2.6 ft WSW of WITNESS POST  15 ft ENE of FOGLINE of I-440 WB  11 ft S of LIGHT POLE  163.0 ft NNW of CENTER of CONCRETE BRIDGE

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note:  These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON		Year: 2016		Point Name: 19-440-31

Latitude(N): 36°07'11.339047"	Longitude(W): 86°48'06.593461"

D.A. Northing:                      651302.2189 (Ft)

D.A. Easting:                       1731783.7350 (Ft)

Elevation:                               582.284 (Ft)

Horz. Datum:  NAD83 (1995)	          		D.A. Factor: 1.00006

Vert. Datum:  NAVD 1988	            	Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:  Aluminum disk atop 5/8” rebar stamped 19-440-31 driven flush

Route Description:  From the junction of I-65 and I-440 south of Nashville, go west on I-440 for 1.7 miles to Mile Marker 3 and the station is on the right.

References:  14.3 ft WSW of WITNESS POST  16.7 ft ENE of FOGLINE of I-440 WB  15.7 ft SSE of MILE MARKER 3  45.0 ft NNW of LIGHT POLE  14.8 ft NW of END of RETAINING WALL

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note:  These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON		Year: 2016		Point Name: 19-440-32

Latitude(N): 36°07'18.620559"	Longitude(W): 86°48'12.968172"

D.A. Northing:                      652042.8328 (Ft)

D.A. Easting:                       1731266.7713 (Ft)

Elevation:                               579.483 (Ft)

Horz. Datum:  NAD83 (1995)	          		D.A. Factor: 1.00006

Vert. Datum:  NAVD 1988	            	Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:  Aluminum disk atop 5/8” rebar stamped 19-440-32 driven flush

Route Description:  From the junction of I-65 and I-440, south of Nashville, go west on I-440 for 1.9 miles to the station on the right.

References:  3 ft WSW of WITNESS POST  17 ft ENE of FOGLINE of I-440 WB  88.0 ft NW of DROP INLET

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note:  These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose

Geodetic Survey Control Data

County: DAVIDSON		Year: 2016		Point Name: 19-440-27

Latitude(N): 36 08'34.63485	             Longitude(W): 86 49'22.44076

D.A. Northing:              659776.9552 (Ft)

D.A. Easting:               1725631.7135 (Ft)

Elevation:                         547.287 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:	1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-27 driven flush

Route Description:  From the junction of I-40 and I-440, west of Nashville, go east on I-440 for 1.0 miles to the station on the right.

References: 2.3 ft E of an orange WITNESS POST  13.8 ft E of a WARNING POST (Buried Cable)  16.3 ft W of the FOGLINE of I-440 exit ramp  41.0 ft NNW of a DROP INLET  55.0 ft S of the center of a CONCRETE BRIDGE

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON		Year: 2016		Point Name: 19-440-28

Latitude(N): 36°08'29.526779"	             Longitude(W): 86°49'23.932377"

D.A. Northing:              659261.4414 (Ft)

D.A. Easting:               1725505.0245 (Ft)

Elevation:                         543.347 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:	1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-28 driven flush

Route Description:  From the junction of I-40 and I-440, west of Nashville, go east on I-440 for 1.1 miles to and onto the Exit 1 ramp and the station on the right.

References: 2.2 ft E of an orange WITNESS POST  12.8 ft W of the FOGLINE of I-440 Exit 1 ramp  17.1 ft SSE of LIGHT POLE #HJ31  25.0 ft NE of a WARNING POST (Buried Post)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: DAVIDSON		Year: 2016		Point Name: 19-440-29

Latitude(N): 36°08'24.043624"	             Longitude(W): 86°49'25.212218"

D.A. Northing:              658707.8530 (Ft)

D.A. Easting:               1725395.3829 (Ft)

Elevation:                         562.631 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:	1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-29 driven flush

Route Description:  From the junction of I-40 and I-440, west of Nashville, go east on I-440 for 1.2 miles to the top of the I-440 Exit 1 ramp at Murphy Rd. and the station on the right.

References: 20.3 ft NE of an orange WITNESS POST at a yield sign  7.5 ft NW of FOGLINE of I-440 Exit 1 Ramp  14.0 ft ESE of a 12 inch REDBUD TREE  41.0 ft NE of a FENCE CORNER  50.2 ft S of a SIGN (East 70S)

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-18

Latitude(N): 36°07'00.49221"	             Longitude(W): 86°46'12.55707"

D.A. Northing:                                650130.2169 (Ft)

D.A. Easting:                                 1741131.1726 (Ft)

Elevation:                               536.608 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-18 driven flush

Route Description:  From the junction of I-440 and I-65, south of downtown Nashville, go east on I-440 for 0.15 mile to the end of guardrail and the station on the right.

References: 6.4 ft NNW of an orange WITNESS POST  13.0 ft SSE of the FOGLINE of I-440  19.4 ft NE of the end of GAURDRAIL

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-20

Latitude(N): 36°07'19.14595"	             Longitude(W): 86°45'12.56158"

D.A. Northing:                   651978.2161 (Ft)

D.A. Easting:                   1746068.1978 (Ft)

Elevation:                             562.781 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-20 driven flush

Route Description:  From the junction of I-440 and I-65, south of downtown Nashville, go east on I-440 for 1.25 miles to Exit 6 (S.R. 11 / US 31A / US 41A / Nolensville Rd.) and the station on the right.

References: 1.5 ft NNW of an orange WITNESS POST  9.3 ft NNW of a CONCRETE DITCH  16.1 ft SSE of the FOGLINE of I-440  111.4 ft ENE of LIGHT POLE # HC08  116.0 ft WSW of LIGHT POLE # HC09

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson		Year: 2011		Point Name: 19-440-21

Latitude(N): 36°07'23.85275"	             Longitude(W): 86°44'57.19716"

D.A. Northing:                    652444.5037 (Ft)

D.A. Easting:                     1747332.3779 (Ft)

Elevation:                              531.837 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-21 driven flush

Route Description: From the junction of I-440 and I-65, south of downtown Nashville, go east on I-440 for 1.5 miles to Exit 6 (S.R. 11 / US 31A / US 41A / Nolensville Rd.) and the station on the right.

References: 13.1 ft SE of the FOGLINE of I-440  16.2 ft NW of an orange WITNESS POST  31.6 ft N of LIGHT POLE # HC16

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.

Geodetic Survey Control Data

County: Davidson 		Year: 2011		Point Name: 19-440-08

Latitude(N):  36°09'05.43959"	             Longitude(W): 86°49'22.55611"

D.A. Northing:              662892.1006 (Ft)

D.A. Easting:               1725648.4459 (Ft)

Elevation:                         619.307 (Ft)

Horz. Datum:  	NAD83(1995)	          D.A. Factor:1.00006

Vert. Datum:   	NAVD 1988	            Geoid Model:  g2003u07

Vertical Order:  GPS

Party Chief: J. Burton		       Described By: J. Burton

Type of Monument:	Aluminum disk atop 5/8” rebar stamped 19-440-08 driven flush

Route Description:  From the junction of I-440 east and I-40 west, just west-southwest of downtown Nashville, go east on I-440 for 0.3 mile to the station on the right.

References: 12.0 ft E of an orange WITNESS POST  13.1 ft E of a CONCRETE WALL  26.3 ft W of the FOGLINE of I-440  77.0 ft SSW of the end of GAURDRAIL

Tennessee Dept. of Transportation - Geodetic Survey Offices

7366 Region Lane, Knoxville, TN 37914

4004 Cromwell Road, Chattanooga, TN 37422

6601 Centennial Blvd, Nashville, TN 37243

300 Benchmark Place, Jackson, TN 38302

Note: These points have been set for the express use of the Tennessee Department of

Transportation and its agents.  No warranty, either express or implied, is granted to any

individual and/or entity that makes use of this data for any other purpose.