TRAFFIC COUNTER SPECIFICATIONS

CONTRACTOR SHALL SUPPLY AND INSTALL ALL RELATED EQUIPMENT SO THAT ONCE A TRAFFIC COUNTER AND MODEM ARE INSTALLED BY THE STATE'S PROJECT PLANNING DIVISION'S  TRAFFIC COUNTER PERSONNEL, THE COUNT STATION WILL BE FULLY OPERATIONAL.

COUNTER CABINET SHALL NOT BE PRE-WIRED FOR A TRAFFIC SIGNAL AND SHALL INCLUDE VENTILATION THROUGH THE TOP AND DOOR BUT DOES NOT TO INCLUDE A VENTILATION FAN.

MINIMUM SIZE OF CABINET SHALL BE 41" TALL, 25" WIDE AND 18" DEEP AND INCLUDE ONE ADJUSTALBE SIDE SHELF. COMPLETE INSTALLATION PER SPECIAL TRAFFIC COUNTER SPECIFICATION NOTES CONCERNING THE COUNTER CABINET. ITEM INCLUDES THE 3" GALVANIZED, RIGID CONDUIT POLE THAT THE CABINET IS TO BE MOUNTED ONTO.

THE CABINET DOOR SHALL BE SEALED WITH A CLOSED CELL NEOPREME GASKET BONDED TO THE INSIDE OF THE DOOR WITH AN OIL RESISTENT ADHESIVE THAT WILL BE POSITIONED TO PREVENT ANYTHING FROM PENTRATING THROUGH THE CRACK BETWEEN THE DOOR AND THE CABINET WHEN THE DOOR IS CLOSED.  CABINET SHALL INCLUDE A THREE POINT LOCKING SYSTEM AND BE MOUNTED SO THAT THE CABINET DOOR WILL OPEN TOWARD THE ROADWAY.

THE CABINET SHALL BE POLE-MOUNTED TO A 3" GALVANIZED, RIGID CONDUIT POLE WITH THE BOTTOM OF THE CABINET AT LEAST 36" ABOVE GROUND LEVEL.  TOP OF THIS POLE SHALL BE AT LEAST EIGHT (8') FEET ABOVE THE GROUND. A 65 WATT SOLAR PANELSHALL BE MOUNTED ON THE 3” RIGID CONDUIT, ALL SOLAR PANEL WIRING SHALL BE RUN THROUGH A WEATHERHEAD AND CONDUIT DOWN THE POLE TO INSIDE THE CABINET AND TERMINATED TO A VOLTAGE REGULATOR.  A 12 VOLT 70-90 AMP HOUR DEEP DISCHARGE MARINE BATTERY SHALL BE INCLUDED.

ALL LOOPS SHALL BE FIVE FEET BY SEVEN FEET (5' X 7') AND TYPICAL IN SIZE WITH THREE (3) TURNS OF LOOP WIRE.

EACH LOOP DETECTOR LEAD-IN (OR SHIELDED DETECTOR CABLE) SHALL BE MARKED WITH LANE IDENTIFICATION, DENOTING THE LANE AND POSITION IN THE LANE FOR EACH LOOP (SUCH AS EB-1 FOR EASTBOUND 1, ECT.).

FOR QUESTIONS CONCERNING ANY OF THE INSTALLATION OF THE COUNTER EQUIPMENT, PLEASE CONTACT STANLEY DUNN AT 615-350-4571.

THE CONTRACTOR SHALL NOTIFY TO REPORT WHEN WORK HAS BEEN COMPLETED SO LONG RANGE PALANNING CAN FINISH THE INSTALLATION.

SPECIAL GENERAL NOTES REQUIRED:

EQUIPMENT AND INSTALLATION OF TRAFFIC SIGNAL ITEMS SHALL COMPLY WITH TDOT STANDARD SPECIFICATIONS, SECTION 730.

DETECTION LOOPS SHALL BE INSTALLED BEFORE THE FINAL SURFACE IS APPLIED.

STANDARD  DRAWINGS REQUIRED: T-SG-2

T-SG-3

ITEMS AND QUANTITIES:

| 730-03.25 | INSTALL PULL BOX (29" X 21") | EACH | 2 |
| --- | --- | --- | --- |
| 730-12.02 | CONDUIT 2" DIAMETER (PVC) | L.F. | 40 |
| 730-14.02 | SAW SLOT | L.F. | 1200 |
| 730-14.03 | LOOP WIRE | L.F. | 4000 |
|  | CABINET | EACH | 2 |

FOOTNOTE ITEM 730-15.07---MINIMUM SIZE OF CABINET SHALL BE 41" TALL, 25" WIDE AND 18" DEEP AND INCLUDE ONE SHELF. COMPLETE INSTALLATION PER SPECIAL TRAFFIC COUNTER SPECIFICATION NOTES CONCERNING THE COUNTER CABINET. ITEM INCLUDES THE 3" GALVANIZED, RIGID CONDUIT POLE THAT THE CABINET IS TO BE MOUNTED ONTO.