SPECIAL TRAFFIC COUNTER SPECIFICATIONS

THE CONTRACTOR SHALL SUPPLY AND INST ALL RELATED EQUIPMENT.  THE CONTRACTOR SHALL MAKE SURE EACH DETECTION LOOP IS OPERATIONAL AT THE COMPLETION OF THE PROJECT.  THE

STATE'S LONG RANGE PLANNING DIVISION'S TRAFFIC COUNTER PERSONNEL WILL MAKE THE TRAFFIC COUNTING SITE FULLY OPERATIONAL.

ALL LOOPS SHALL BE 5 FOOT BY 7 FOOTAND TYPICAL IN SIZE WITH THREE (3) TURNS OF LOOP WIRE.

LEAD WIRES TO ALL LOOPS SHALL BE RUN THROUGH 2 INCH CONDUIT FROM THE SHOULDER OF THE ROADWAY TO AN IN-GROUND PULL BOX. THE PULL BOX SHOULD BE LOCATED AS CLOSE AS POSSIBLE AND BEHIND THE GUARDRAIL.  LOOP LEAD-INS AND CONDUIT SHALL BE MARKED ON THE ROADWAY SURFACE SO THAT ANY GUARDRAIL POST INSTALLATION OR OTHER CONSTRUCTION WILL NOT DISTURB THIS UNDERGROUND WORK.

AN EXTRA 20 FEETOF LOOP WIRE FOR EACH LOOP SHALL BE STORED IN THE PULL BOX, EACH LOOP DETECTOR LEAD-IN SHALL BE MARKED WITH LANE IDENTIFICATION, DENOTING THE LANE AND POSITION IN THE LANE FOR EACH LOOP (SUCH AS NB-1 FOR NORTH BOUND ETC.).

PLEASE CONTACT STANLEY DUNN AT 615-350-4571 WHEN THE LOOPS AND PULL BOXES ARE INSTALLED OR IF ANY QUESTIONS OCCUR DURING THE INSTALLATION OF LOOPS OR PULL BOXES.

SPECIAL GENERAL NOTES REQUIRED:

EQUIPMENT AND INSTALLATION OF TRAFFIC SIGNAL ITEMS SHALL COMPLY WITH TDOT STANDARD SPECIFICATIONS, SECTION 730.

DETECTION LOOPS SHALL BE INSTALLED BEFORETHE FINAL SURFACE  IS APPLIED.