TDOT WORK ZONE SAFETY AND MOBILITY MANUAL

TMP Workbook

Work Zone Significance Determination

| State PE Number: |  |  |  | Route/From-To: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PIN: |  |  |  | County: |  |  |  |
| Analyst: |  |  |  | Project/Construction AADT: |  |  |  |

This is an	Initial     Secondary      determination of the project’s significance.

Major Route Criteria

| A project lasting at least three days on an interstate route within a TMA with intermittent or continuous lane closures |  |  |
| --- | --- | --- |
| A project where all lanes in one direction will be closed on (a) any interstate route or (b) a non-interstate route having an AADT of at least 50,000 vpd |  |  |
|  | Yes, by the Major Route Criteria, this is a Significant Project. |  |
|  | No, the Major Route Criteria are not met. |  |

Delay Criteria

| Urban |  |  | Rural |  | Freeway |  |  | Arterial |  | Collector/Other |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of lanes (in one direction) to be open in work zone: |  |  |  |  |  |  |  | Max. Allowable AADT (24-hr, two-way) from Table 3.1: |  |  |  |  |  |
|  | Yes, by the Delay Criteria, this is a Significant Project (project AADT > max AADT). |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No, the Delay Criteria are not met (project AADT < max AADT). |  |  |  |  |  |  |  |  |  |  |  |  |

Qualitative Criteria

Rate the following aspects of the work zone:

|  |  |  |  |  |  | High | Low |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Business impacts (how many businesses affected?) |  |  |  |  |  |  |  |  |
|  | Public Interest |  |  |  |  |  |  |  |  |
|  | Exposure impacts due to long project duration |  |  |  |  |  |  |  |  |
|  | Impacts due to alternate routes |  |  |  |  |  |  |  |  |
|  | Impacts due to other concurrent projects nearby |  |  |  |  |  |  |  |  |
|  |  | Concurrent project description: |  |  |  |  |  |  |  |
|  | Other: |  |  |  |  |  |  |  |  |
|  |  |  | Yes, due to extraordinary Qualitative Criteria, this is a Significant Project.* |  |  |  |  |  |  |
|  |  |  | No, the Qualitative Criteria are not met. |  |  |  |  |  |  |
| * “Significance” based solely on Qualitative criteria to be carefully considered and approved by responsible Division Director |  |  |  |  |  |  |  |  |  |

FINAL TMP DETERMINATION:  	Significant Project:   	Yes  	No

TMP Exception

| Per FHWA/TDOT guidelines, an EXCEPTION has been applied for and approved by the FHWA Division Office (attach appropriate documentation) |  |
| --- | --- |

| Manager I – Division Level |  | Date |
| --- | --- | --- |
| Manager II – Division Level |  | Date |

Transportation Management Plan

| Project Description: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| State PE Number: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PIN: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| County: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project Determined As (Refer to Figure 3.1 and the following sheets for guidance) : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Check One) Basic |  |  |  |  |  |  |  |  |  | Intermediate |  |  | Significant |
| TMP Description (Check each Component utilized in Project) : |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Temporary Traffic Control: (If Basic is selected above, only TTC strategies will be used) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Transportation Operations: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Public Information: |  |  |  |  |  |  |  |  |  |  |  |  |
| TMP Prepared By (Division/Firm Name): |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TDOT Manager I: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Phone Number: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Email Address: |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDOT Manager II Approval: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Title: |  |  |  |  |  |  |  |  |  |  | Division: |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |

Statement of Categories of TMPs

Significant Project - Requires careful consideration of work zone impact mitigation.   Requires use of strategies from all three of TMP categories to help mitigate the impacts of a significant project:

- Temporary Traffic Control Strategies (TTC)

- Transportation Operations Strategies (TO)

- Public Information Strategies (PI)

A Significant Project is one for which any of the following criteria exists:

- Any project on the interstate system located within a recognized Transportation Management Area (TMA) that occupies a given location for at least three days duration with either continuous or intermittent lane closures.  For the purposes of this Manual, it is assumed that the TMA consists of the following counties: Blount, Bradley, Carter, Davidson, Fayette, Grainger, Hamblen, Hamilton, Hawkins, Jefferson, Knox, Loudon, Madison, Maury, Montgomery, Robertson, Rutherford, Sevier, Shelby, Sullivan, Sumner, Washington, Williamson, Wilson.

- Any project of any duration on an interstate route, or any other route with an ADT of at least 50,000 vehicles per day for which all lanes in one direction will be closed to traffic.

- Any project for which the delay through the limits of the work zone is at least 30 minutes above the normal delay under typical non-work conditions.

- Any project deemed Significant by extraordinary qualitative characteristics. This determination may be made on the basis of conditions such as high levels of public interest, business/community impacts, or long work zone duration. All Significant Projects defined in this manner shall only be done with careful consideration and strategic decision making.

For a “non-significant” Project, one of two different levels of TMPs may be developed. A Basic TMP is to be used when only a TTC plan is needed to successfully implement a safe and efficient work zone. An Intermediate TMP is to be completed for “non-significant” projects where public information and/or transportation operation strategies would be beneficial or necessary as determined based on TDOT decision-making practices and procedures.

Intermediate Project - Requires additional planning, coordination, etc., but not required to be at the same level of a Significant TMP.  TMP requires use of PI and/or TO strategies beyond the project’s TTC strategies. It is expected that the majority of Intermediate projects will consist of TTC and PI strategies.

Basic Project - Typical work zone TTC plan is implemented alone. Refer to TDOT standard drawings, standard notes, and MUTCD.  No additional TMP strategies required.

Temporary Traffic Control Strategies

The Temporary Traffic Control component of the Transportation Management Plan (TMP) is included in the Contract Documents.  The various strategies described below are intended for use in developing the final TMP.  Although many of these strategies are routinely included in the Temporary Traffic Control Plan, they are provided here for review and consideration.

- Construction phasing and/or equipment staging

- Full roadway closure

- Narrow lane/shoulder widths to maintain existing number of lanes

- Full closure of lane/shoulder

- Lane shift to shoulder/median

- One-lane, two-way operation

- Two-way traffic on one side of divided roadway

- Reversible lanes

- Ramp closure/relocation

- Directional interchange closure

- Work hour restrictions (off-peak, night, weekend)

- Bike/Ped access maintenance

- Private property/business access maintenance

- Off-site detour/alternate routes

- Temporary guidance/informational signs

- Portable changeable message signs

- Flashing arrow boards

- Flaggers/uniformed traffic control personnel

- Temporary traffic signals

- Warning lighting devices

- Coordination with other construction projects

- Coordination with other utility projects

- Coordination of existing/future right-of-way needs

- Coordination with other non-highway transportation facilities

- Incentive/disincentive clauses

- Innovative construction techniques/materials

The various TTC Strategies listed above are not intended to be all inclusive, but rather provide a range of possible strategies which might be considered by those persons developing the TMP.  Use the space below (and/or attach additional pages) to highlight noteworthy strategies OR strategies which are not routinely used on this type of project.

Notes for Selection of TTC Strategies:

Transportation Operations Strategies

The Transportation Operations Strategies component of the Transportation Management Plan (TMP) is included in the Contract Documents.  The various strategies described below are intended for use in developing the final TMP.  Although many of these strategies are routinely included, they are provided here for review and consideration.

- Transit service additions/improvements

- Transit incentives

- Shuttle services

- Ridesharing/carpooling incentives

- Park-and-ride promotion

- HOV lane addition/promotion

- Ramp metering

- Variable work hour incentives

- Signal timing/coordination improvements

- Temporary traffic signals

- Other street/intersection improvements

- Bus/delivery turnouts

- Turn restrictions

- Parking restrictions

- Truck/heavy vehicle restrictions

- Separate truck lanes

- Reversible lanes

- Ramp closures

- Railroad crossing controls

- Speed limit reduction/variable speed limits

- Temporary movable/traffic barrier system

- Crash cushion

- Temporary rumble strips

- Intrusion alarm

- Warning lights

- Automated flagger assistance devices

- Road safety audits

- On-site safety training

- Safety awards/incentives

- Windshield safety surveys

- ITS for traffic monitoring/management

- Transportation management centers

- Traffic surveillance

- Traffic screens

- Assistance call boxes

- Temporary location mile markers

- Tow/freeway service patrol

- Incident detour routing

- Contract support for incident management

- Incident/emergency response plan

- Dedicated (paid) police enforcement

- Cooperative police enforcement

- Automated enforcement

- Aerial enforcement

- Project task force/committee

- Construction safety supervisor/inspector

- Incident/emergency management coordinator

- TMP monitor/inspection team

- Team meetings

The various Transportation Operations Strategies listed above are not intended to be all inclusive, but rather provide a range of possible strategies which might be considered by those persons developing the TMP.  Use the space below (and/or attach additional pages) to highlight noteworthy strategies OR strategies which are not routinely used on this type of project.

Notes for Selection of TO Strategies:

Public Information Strategies

The Public Information component of the Transportation Management Plan (TMP) is coordinated by the Department’s Communications Office.  The Department employs a wide range of standard public information related strategies on all relevant projects.  These include, but are not limited to, the 511 Travel Information System, the Dynamic ITS Message Boards, and the TDOT Internet Site.

In addition, lane-closure meetings are held weekly where additional public information strategies are considered and initiated when warranted.

All work zone projects will follow the guidelines set forth in TDOT’s Public Involvement Plan.  The various strategies described below are intended for use in developing the final TMP and are provided here for review and consideration.

- Brochures and mail-outs

- Press releases/media alerts

- Paid advertisements

- Public information center

- Telephone hotline

- General TDOT website

- Specific project website

- Public meetings/hearings

- Community task force

- Coordination with media/schools/businesses/etc

- Work zone education/safety campaign

- Visual presentation materials

- Traffic radio broadcasts

- Advanced placement, changeable message signs

- Temporary motorist information signs

- Dynamic speed message sign

- Highway advisory radio (HAR)

- Listing on Tennessee 511

- Freight information

The various Public Information Strategies listed above are not intended to be all inclusive, but rather provide a range of possible strategies which might be considered by those persons developing the TMP.  Use the space below (and/or attach additional pages) to highlight noteworthy strategies OR strategies which are not routinely used on this type of project.

Notes for Selection of PI Strategies:

| Manager I –Requesting PI Input |  | Date |  |
| --- | --- | --- | --- |
| Community Relations Office – (To be sent back to Manager I above) |  | Date |  |

ADDITIONAL NOTES/DOCUMENTATION